Soleux Control API

TCP Default + 3 and HTTP/HTTPS Command Specification

Status: implemented protocol 2 command catalogue for all five active device profiles

Audience: Device firmware, Windows application and mobile application developers

Transports: TCP legacy port + 3 (5008 by default), HTTP port 80 and HTTPS port 443

Compatibility: migrated device base ports are AT-only; Windows device paths use the Control API

Implementation status: Relay Module, AC/DC Dimmer, PDU Energy Meter, High Power PDU and PDU V1 firmware expose the implemented protocol 2 command subsets on the configured base TCP port plus 3 and through POST /api/v1/command over HTTP or HTTPS. This specification lists only those implemented actions; device-specific actions remain capability-gated.

# Document purpose

This specification defines the implemented transport-neutral command model for Soleux device firmware, the Windows application and mobile clients. Migrated device control uses the dedicated Control API transport; the configured base port remains available for legacy AT clients.

## Normative rules

- All physical input, virtual-input and output indexes are zero-based on every transport.

- JSON booleans are true/false; API clients must not send 0/1 as boolean substitutes.

- Time durations use integer milliseconds unless a field name explicitly ends in _s.

- Electrical and environmental units are included in field names: _v, _a, _w, _va, _wh, _hz and _c.

- Timestamps use ISO 8601 with an offset. Event timestamps should be UTC where practical.

- Unknown fields may be ignored only when the negotiated protocol version permits forward-compatible extensions.

- Secrets are write-only unless a command explicitly documents that they can be returned.

- Every state-changing command identifies its required permission and produces an auditable source/session identity.

## Command groups

- Session and identity: 4 implemented actions.

- Core device state: 4 implemented actions.

- Input control: 3 implemented actions on supported profiles.

- Output control: 3 implemented actions on all profiles.

- Mapping and hardware control: 2 implemented actions on supported profiles.

- Dimmer control and output groups: 6 implemented actions on the AC/DC Dimmer.

- Energy metering: 2 implemented actions on the PDU Energy Meter.

- Configuration pages: 4 implemented actions on all profiles.

- Transfers: 7 implemented actions on all profiles.

Catalogue size: 31 implemented request actions.

# Transport and message envelope

The action and params object is identical on TCP, HTTP and HTTPS. Each HTTP request is dispatched through the same command handler used by the newline-delimited TCP service, so command parameters and result envelopes do not change between transports.

## Request envelope

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| protocol | integer | Yes | 2 for current Relay; 3 for target contract | Protocol version. |
| id | integer¦string | Yes | Unique among pending requests | Caller-generated correlation ID. |
| action | string | Yes | Exact action name | Command to execute. |
| params | object | No | Default {} | Command-specific parameters. |
| timeout_ms | integer | No | 100-120000 | Optional caller timeout hint; does not override safety limits. |

{"protocol":2,"id":42,"action":"set_output_state","params":{"channel":0,"state":true}}

## Success response

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| protocol | integer | Always | Protocol version. |
| id | integer¦string | Always | Original request ID. |
| ok | boolean | Always | True for a successful command. |
| result | object | Always | Command-specific expected result. |

{"protocol":2,"id":42,"ok":true,"result":{"channel":0,"requested_state":true,"actual_state":true,"pending":false}}

## Error response

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| protocol | integer | Always | Protocol version. |
| id | integer¦string¦null | Always | Original ID, or null when it could not be parsed. |
| ok | boolean | Always | False for an error. |
| error.code | string | Always | Stable machine-readable error code. |
| error.message | string | Always | Safe human-readable explanation. |
| error.details | object | Optional | Structured validation or recovery data. |
| error.retryable | boolean | Always | Whether retry may succeed without changing the request. |

{"protocol":2,"id":42,"ok":false,"error":{"code":"invalid_request","message":"output channel out of range"}}

## Transport mapping

| Transport | Request framing | Response framing | Events |
| --- | --- | --- | --- |
| TCP: legacy port + 3 | One UTF-8 JSON object per line (CRLF or LF). Default 5005 becomes Control API 5008. | One JSON response line using the common envelope. | Command responses only in the current Relay implementation. |
| HTTP port 80 | POST /api/v1/command with application/json. | HTTP status plus the common JSON envelope. | No Control API SSE endpoint is implemented yet. |
| HTTPS port 443 | Same endpoint over TLS. When SSL is enabled, HTTP receives a 307 redirect that preserves POST and body. | Same common JSON envelope. | No secure event endpoint is implemented yet. |
| WebSocket / SSE | Reserved for a future event transport. | Not currently a Control API command transport. | Planned; capability-dependent. |

## Implemented HTTP/HTTPS endpoint

Each migrated device family exposes one transport-neutral command endpoint. Every action available on that device's JSON TCP Control API is accepted at POST /api/v1/command, including state/configuration operations, page configuration and actions, system-log pagination and deletion, and supported file-transfer operations.

| Property | Implemented behavior |
| --- | --- |
| Endpoint | POST /api/v1/command |
| Media type | Request and response use application/json; the response also sets Cache-Control: no-store. |
| Request body | One JSON object using the common protocol, id, action and params fields. |
| Maximum body | 1,048,576 bytes. The limit is enforced with or without a Content-Length header. |
| Dispatch | The request is passed to the same process_tcp_json dispatcher used by the TCP Control API. |
| Client access | api_enabled must be true. Existing HTTP API IP/MAC rules then apply to the direct socket peer; forwarding headers are not trusted for authorization. |
| HTTP/HTTPS | The same Flask endpoint is served on HTTP 80 when SSL is off and HTTPS 443 when SSL is on. |
| Legacy API | The existing /cmdapi endpoint is unchanged and remains separate from this JSON command endpoint. |

## HTTP status mapping

| Status | Meaning | Response |
| --- | --- | --- |
| 200 | The command completed successfully. | ok is true and result contains the command output. |
| 400 | Malformed body, invalid parameters or unknown action. | ok is false with invalid_request, unknown_action or another command error. |
| 403 | The peer is not allowed by the configured API IP/MAC rules. | ok is false with unauthorized. |
| 413 | The request body exceeds 1 MiB. | ok is false with message_too_large. |
| 503 | The HTTP/HTTPS Control API is disabled. | ok is false, id is null and error.code is api_disabled. |
| 504 | An asynchronous device or hardware operation timed out. | ok is false with timeout. |
| 500 | The dispatcher failed internally or produced an invalid response. | ok is false with internal_error. |

## HTTP request example

POST /api/v1/command HTTP/1.1
Content-Type: application/json

{"protocol":2,"id":"mobile-1","action":"hello","params":{}}

{"protocol":2,"id":"mobile-1","ok":true,"result":{"device":"dimmer","api_port":5008}}

## Verification commands

Run the source-level contract tests from the Soleux App project root on a developer computer with Python 3. Each active device family has its own tests folder; the Dimmer command contract is verified with:

python -m unittest discover -s "Device Firmware\Dimmer\tests" -p "test_*.py" -v

After deploying the matching firmware files and restarting WebService, run this on the device to verify the live HTTP endpoint:

curl -sS -X POST http://127.0.0.1/api/v1/command \
  -H 'Content-Type: application/json' \
  -d '{"protocol":2,"id":"test-1","action":"hello","params":{}}'

Use https://127.0.0.1/api/v1/command when SSL is enabled. A successful response has ok: true and reports the calculated api_port.

Compatibility rule: The J: prefix is not accepted on the Control API or migrated base ports. Standard Windows paths use unprefixed JSON on base port + 3 and do not send AT commands or fall back to the AT-only base port. Adding the HTTP/HTTPS endpoint does not change existing legacy AT behavior.

# Permissions and security

| Permission | Allows |
| --- | --- |
| Public | hello only. |
| Read | Read identity, state, configuration summaries, telemetry and events. |
| Control | Change live input/output/dimmer state and execute schedules/scenarios. |
| Configure | Change mappings, channel configuration, schedules, automations and non-secret settings. |
| Admin | Accounts, access rules, network/security settings, backup/restore, firmware, reboot and destructive actions. |

The implemented HTTP/HTTPS command endpoint first requires api_enabled. When enabled, each device's existing API IP/MAC allow rules apply to the direct socket peer. Disabling filtering allows any reachable peer; it is different from disabling the API service. Forwarding headers are not trusted. Restrict HTTP to trusted local networks and use HTTPS for credentials, backup/restore, firmware, or remote/mobile access. Tokens and secrets must never be written to normal logs.

## Implemented service access switches

All five active firmware families persist the following independent Security-page switches. Both default to enabled. Changing either switch schedules a WebService restart after the current response is delivered.

| Security field | Behavior when disabled | Still available and recovery |
| --- | --- | --- |
| tcp_enabled | Closes and suppresses the base TCP, base + 1 and Control API TCP base + 3 listeners after restart. Existing TCP sessions close and the Windows app cannot connect. | UDP discovery, UDP heartbeat base + 2 and the web UI remain available. Re-enable TCP from the web Security page. |
| api_enabled | HTTP/HTTPS command routes return status 503 with id null and error.code api_disabled. Raw Control API TCP is not closed. | The normal web UI remains available. Re-enable API from the web Security page. |

Disabling API does not close Control API TCP; disabling TCP does. If both are disabled, the web Security page remains the recovery path. Discovery or heartbeat success therefore does not prove that device control is enabled.

{"protocol":2,"id":null,"ok":false,"error":{"code":"api_disabled","message":"HTTP/HTTPS API access is disabled."}}

## Implemented AC/DC Dimmer profile

The active Dimmer firmware accepts unprefixed protocol 2 JSON on base_port + 3 (normally 5008) and the identical command object through POST /api/v1/command over HTTP or HTTPS. The base port remains AT-only and rejects J-prefixed traffic.

All Dimmer input, output and mapping indices are zero-based. set_dimmer_level requires channel and an integer value from 0 through 100. set_output_state and restart_output use the same zero-based channel rule. Output-group master and local-slave channels are also zero-based; remote output_number values are one-based. A remote slave stores its Dimmer base port, and PWM synchronization uses that port plus 1.

get_device_state returns requested set_pwm and live actual_pwm for every output, plus I/O state, PWM frequency, sensors, system and network data. Clients should poll this snapshot when a state-changing result reports pending: true.

The Dimmer also implements device identity, relay configuration, input/output configuration, mappings, hardware write, output groups, settings/security/system/schedule/automation/scenario/sequence pages, chunked backup/restore, paginated system logs and execute_page_action delete_system_logs.

Discovery advertises PORT, API_PORT, API_VER and CAPS. Heartbeat uses base_port + 2 and reports tcp_port, api_port and api_version. Both offsets are calculated from the configured base port.

# Common error catalogue

Every error uses the common error envelope. Commands may document additional codes.

| Code | Meaning |
| --- | --- |
| invalid_request | Envelope or JSON structure is invalid. |
| unknown_action | Action name is not recognized. |
| unsupported_command | Device recognizes the action but does not support it. |
| unsupported_protocol | No mutually supported protocol version exists. |
| authentication_required | A valid authenticated session is required. |
| authentication_failed | Credentials or token were rejected. |
| permission_denied | Session lacks the required permission. |
| invalid_parameter | A parameter has the wrong type, range or value. |
| invalid_channel | Input/output channel is outside the zero-based device range. |
| not_found | Requested resource or operation does not exist. |
| revision_conflict | expected_revision does not match current resource revision. |
| busy | A conflicting operation is active. |
| interlock | A safety or control interlock prevented the operation. |
| timeout | The operation did not complete within its allowed time. |
| rate_limited | Caller exceeded the configured request limit. |
| payload_too_large | Message or transfer exceeds advertised limits. |
| internal_error | Unexpected device-side failure; message must not expose secrets. |
| api_disabled | The HTTP/HTTPS Control API is disabled in device Security settings. |

# Implemented command catalogue

This catalogue contains only request actions implemented by the active protocol-2 firmware profiles. Device-specific actions are listed with their applicable profile; clients should still use the hello capability list before sending a device-specific action.

| Action | Implemented by | Parameters | Result or purpose |
| --- | --- | --- | --- |
| hello | All active profiles | None | Protocol, identity and capability list. |
| get_device_info | All active profiles | None | Stable identity, firmware and network information. |
| get_device_state | All active profiles | None | Live inputs, outputs, PWM, sensors, system and network snapshot. |
| get_relay_configuration | All active profiles | None | Input, output and mapping configuration. |
| set_input_state | Relay, Dimmer, PDU Energy, PDU 10 kW | channel, state, source | Inject a physical input state. |
| set_virtual_input_state | Relay, Dimmer, PDU Energy, PDU 10 kW | channel, state, source | Inject a virtual input state. |
| set_input_configuration | Relay, Dimmer, PDU Energy, PDU 10 kW | channel, name, enabled | Save one input configuration. |
| set_output_state | All active profiles | channel, state, source | Request an output on or off state. |
| restart_output | All active profiles | channel, source | Restart one output. |
| set_output_configuration | All active profiles | Device-specific output fields | Save one output configuration. |
| set_mapping | Relay, Dimmer, PDU Energy, PDU 10 kW | input, output, code | Save one input-to-output mapping. |
| write_hardware | Relay, Dimmer, PDU Energy, PDU 10 kW | None | Publish saved mappings and output delays to hardware services. |
| set_dimmer_level | AC/DC Dimmer | channel, value (0-100) | Set one PWM brightness level. |
| set_dimmer_frequency | AC/DC Dimmer | value | Save and apply the dimmer frequency mode. |
| get_output_groups | AC/DC Dimmer | None | Read Dimmer output names and all local/remote output groups. |
| save_output_group | AC/DC Dimmer | group | Create or update one output group and publish its local mask. |
| delete_output_group | AC/DC Dimmer | group_id | Delete one output group and publish the revised local masks. |
| rewrite_output_groups | AC/DC Dimmer | None | Rewrite every local group mask to the MCU and wait for its acknowledgement. |
| get_energy_state | PDU Energy Meter | None | Read current aggregate energy measurements. |
| get_energy_history | PDU Energy Meter | parameter, start_date, end_date | Read stored energy history. |
| get_page_configuration | All active profiles | page, optional pagination | Read a supported configuration page. |
| set_page_configuration | All active profiles | page, section, values | Save a supported configuration page section. |
| mutate_page_row | All active profiles | page, section, operation, row_id, values | Create, update or delete a supported page row. |
| execute_page_action | All active profiles | page_action and action-specific fields | Execute supported system, time, log and reboot actions. |
| transfer_upload_begin | All active profiles | kind, role, filename, size | Start a chunked upload session. |
| transfer_upload_chunk | All active profiles | transfer_id, role, offset, data | Append one upload chunk. |
| transfer_upload_finish | All active profiles | transfer_id, role | Finish and validate an upload. |
| transfer_commit | All active profiles | transfer_id | Commit a validated upload or restore. |
| settings_backup_download_begin | All active profiles | None | Start a settings-backup download. |
| transfer_download_chunk | All active profiles | transfer_id, offset | Read one download chunk. |
| transfer_download_finish | All active profiles | transfer_id | Finish a download session. |

The catalogue intentionally excludes target or proposed actions that do not appear in the active firmware dispatcher and hello capability data.

# 1. Device session and state

## 1.1  hello

Negotiate protocol 2 and return the device identity, installed channel counts, API port and advertised capabilities.

Implemented by: All active profiles    Operation: Public read

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| protocol | integer | Always | Current Control API protocol number, 2. |
| device | string | Always | Device profile identifier. |
| name | string | Always | Configured device name. |
| input_count | integer | Always | Installed physical/control input count. |
| virtual_input_count | integer | Always | Installed virtual input count, or zero. |
| output_count | integer | Always | Installed controllable output count. |
| api_port | integer | Always | Calculated Control API TCP port. |
| capabilities | string[] | Always | Advertised action and feature capabilities. |

Command errors: invalid_request, internal_error

## 1.2  get_device_info

Read stable device identity, firmware/build information and Ethernet/Wi-Fi MAC addresses.

Implemented by: All active profiles    Operation: Read

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| device_type | string | Always | Device family/profile. |
| model | string | Device dependent | Hardware model or product name. |
| serial_number | string | Always | Configured device serial number. |
| firmware_version | string | When available | Installed firmware version. |
| build_number | string | When available | Installed build identifier. |
| release_date | string | When available | Firmware release/build date. |
| network | object | Always | Ethernet and Wi-Fi MAC addresses. |

Command errors: internal_error

## 1.3  get_device_state

Read the complete live snapshot used by desktop and mobile clients for synchronization.

Implemented by: All active profiles    Operation: Read

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| revision | integer | When available | Snapshot revision/timestamp value. |
| captured_at | datetime | When available | Snapshot capture time. |
| inputs | object[] | Profiles with inputs | Current zero-based input states. |
| outputs | object[] | Always | Output names, states and device-specific live values. |
| sensors | object[] | When available | Temperature and environmental readings. |
| system | object | Always | Time, uptime, memory, CPU and temperature data. |
| network | object | Always | Current LAN/Wi-Fi addresses and SSID. |
| energy | object | PDU Energy Meter | Current electrical measurements. |
| manual_override_active | boolean | PDU profiles | Current manual override state. |

Command errors: internal_error

## 1.4  get_relay_configuration

Read the installed input/output configuration and input-to-output mapping matrix.

Implemented by: All active profiles    Operation: Read

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| input_count | integer | Always | Installed input count. |
| output_count | integer | Always | Installed output count. |
| inputs | object[] | Always | Input names, enable flags and profile fields. |
| outputs | object[] | Always | Output names, delays, interlocks and profile fields. |
| mapping | object[] | When supported | Zero-based input/output mapping data. |

Command errors: internal_error

# 2. Input output and mapping control

## 2.1  set_input_state

Inject a zero-based input state through the same control path used by the device input service.

Implemented by: Relay, Dimmer, PDU Energy, PDU 10 kW    Operation: Control

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to input_count - 1 | Zero-based input channel. |
| state | boolean | Yes | true or false | Requested input state. |
| source | string | No | Default Control API; max 64 chars | Audit/source description. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Accepted input channel. |
| state | boolean | Always | Requested input state. |
| accepted | boolean | Always | True when queued/published. |

Command errors: invalid_request, internal_error

## 2.2  set_virtual_input_state

Compatibility alias for input-state injection on profiles that expose virtual input control.

Implemented by: Relay, Dimmer, PDU Energy, PDU 10 kW    Operation: Control

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to input_count - 1 | Zero-based input channel in the profile input space. |
| state | boolean | Yes | true or false | Requested virtual input state. |
| source | string | No | Default Control API; max 64 chars | Audit/source description. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Accepted input channel. |
| state | boolean | Always | Requested input state. |
| accepted | boolean | Always | True when queued/published. |

Command errors: invalid_request, internal_error

## 2.3  set_input_configuration

Save the editable configuration for one zero-based input channel.

Implemented by: Relay, Dimmer, PDU Energy, PDU 10 kW    Operation: Configure

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to input_count - 1 | Input channel to update. |
| name | string | No | Profile length rules | New display name. |
| enabled | boolean | No | true or false | Whether the input is enabled. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| input | object | Always | Updated input configuration object, returned directly or as the command result. |

Command errors: invalid_request, internal_error

## 2.4  set_output_state

Request one output to turn on or off while enforcing manual-override and configured interlocks.

Implemented by: All active profiles    Operation: Control

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to output_count - 1 | Zero-based output channel. |
| state | boolean | Yes | true or false | Requested output state. |
| source | string | No | Default Control API | Audit/source description when supported. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Controlled output channel. |
| requested_state | boolean | Always | Requested logical state. |
| actual_state | boolean | Always | Latest known hardware state. |
| pending | boolean | Always | True until hardware feedback matches the request. |

Command errors: invalid_request, internal_error

## 2.5  restart_output

Start the existing output restart operation for one zero-based output channel.

Implemented by: All active profiles    Operation: Control

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to output_count - 1 | Output channel to restart. |
| source | string | No | Default Control API | Audit/source description. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Restarted output channel. |
| accepted | boolean | Always | True when the restart was queued. |
| operation_id | string | Always | Identifier for the asynchronous restart. |
| off_time_ms | integer | Dimmer | Configured restart off-time used by the Dimmer path. |

Command errors: invalid_request, internal_error

## 2.6  set_output_configuration

Save the editable configuration for one output; supported fields vary by device profile.

Implemented by: All active profiles    Operation: Configure

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to output_count - 1 | Output channel to update. |
| name | string | No | Profile length rules | New display name. |
| enabled | boolean | No | Profile dependent | Whether the output is enabled. |
| delay/runtime/interlock fields | mixed | No | Profile dependent | On/off delay, runtime, restart-disable, turn-off-disable and Dimmer PWM fields. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| output | object | Always | Updated output configuration object, returned directly or as the command result. |

Command errors: invalid_request, internal_error

## 2.7  set_mapping

Save one zero-based input-to-output mapping using the current numeric behavior code.

Implemented by: Relay, Dimmer, PDU Energy, PDU 10 kW    Operation: Configure

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| input | integer | Yes | 0 to input_count - 1 | Input side of the mapping. |
| output | integer | Yes | 0 to output_count - 1 | Output side of the mapping. |
| code | integer | Yes | Profile mapping-code range | Existing device mapping behavior code. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| saved | boolean | Always | True after the database update was accepted. |

Command errors: invalid_request, internal_error

## 2.8  write_hardware

Publish saved mappings and output-delay configuration to the hardware-control services.

Implemented by: Relay, Dimmer, PDU Energy, PDU 10 kW    Operation: Configure

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| hardware_write_started | boolean | Relay/PDU profiles | True when the asynchronous write begins. |
| hardware_write_complete | boolean | Dimmer | True when the Dimmer write request completes. |

Command errors: internal_error

# 3. Dimmer and energy control

## 3.1  set_dimmer_level

Set the PWM brightness percentage for one Dimmer output channel.

Implemented by: AC/DC Dimmer    Operation: Control

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to output_count - 1 | Zero-based Dimmer output channel. |
| value | integer | Yes | 0 through 100 | Requested PWM percentage. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Controlled Dimmer channel. |
| requested_pwm | integer | Always | Requested PWM percentage. |
| actual_pwm | integer | Always | Latest known hardware PWM percentage. |
| pending | boolean | Always | True while actual PWM differs from requested PWM. |

Command errors: invalid_request, internal_error

## 3.2  set_dimmer_frequency

Save and apply the AC/DC Dimmer frequency-mode value.

Implemented by: AC/DC Dimmer    Operation: Configure

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| value | integer | Yes | AC: 0 or 1; DC: 1, 2 or 3 | Frequency mode accepted by the installed Dimmer type. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| pwm_frequency | integer | Always | Saved frequency-mode value. |
| saved | boolean | Always | True after persistence and publish. |

Command errors: invalid_request, internal_error

## 3.3  get_output_groups

Read the AC/DC Dimmer Output Groups configuration, including the available local outputs and every saved group. Each group reports whether it is enabled and its remote retry, timeout, and failure settings. Local channels are zero-based; each remote output number is one-based.

Implemented by: AC/DC Dimmer    Operation: Read

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| page | string | Always | output_groups. |
| device_family | string | Always | dimmer. |
| output_count | integer | Always | Number of configured local Dimmer outputs. |
| outputs | object[] | Always | Local channel and output_name entries. |
| groups | object[] | Always | Saved group objects with local and remote slaves, enabled, remote_retry_count, remote_timeout_ms and remote_failure_action. |

Command errors: internal_error

## 3.4  save_output_group

Create or update one AC/DC Dimmer output group. The save commits the database change and publishes the complete local group-mask table to the MCU service. A remote slave stores the remote Dimmer's configured base port; PWM synchronization is sent to that port plus 1.

Implemented by: AC/DC Dimmer    Operation: Configure

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| group | object | Yes | Required | Output group to create or update. |
| group.id | integer or null | No | Existing positive ID; omit/null to create | Saved group identifier. |
| group.name | string | Yes | 1 to 64 characters | Unique group name, compared case-insensitively. |
| group.master_channel | integer | Yes | 0 to output_count - 1 | Zero-based local master output; unique across groups. |
| group.enabled | boolean | No | Default true | Enables or disables local MCU synchronization and remote Dimmer delivery for this group. |
| group.remote_retry_count | integer | No | 0 to 10; default 2 | Number of extra TCP connection/send attempts after the first attempt for each remote slave. |
| group.remote_timeout_ms | integer | No | 100 to 10000; default 1000 | TCP connect/send timeout in milliseconds for each remote attempt. |
| group.remote_failure_action | string | No | retry_next_feedback or hold_until_master_changes | Failure policy after all remote attempts fail. retry_next_feedback clears the cached value so the next MCU report retries it; hold_until_master_changes waits for a different master value. |
| group.local_slaves | integer[] | Yes | Zero-based local outputs | Local slaves; must not contain the master. |
| group.remote_slaves | object[] | Yes | Remote Dimmer outputs | Remote slaves. A group needs at least one local or remote slave and permits at most 32 total slaves. |
| remote.name | string | No | 0 to 64 characters | Display name for a remote slave. |
| remote.ip_address | IPv4 string | Conditional | Valid IPv4 address | Remote Dimmer address; required for each remote slave. |
| remote.port | integer | Conditional | 1 to 65534 | Remote Dimmer base device port; required for each remote slave. |
| remote.output_number | integer | Conditional | 1 to 16 | One-based remote Dimmer output; required for each remote slave. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| page | string | Always | output_groups. |
| device_family | string | Always | dimmer. |
| output_count | integer | Always | Number of configured local Dimmer outputs. |
| outputs | object[] | Always | Refreshed local output list. |
| groups | object[] | Always | Refreshed saved output groups, including enabled and remote failure settings. |
| saved_group_id | integer | Always | Created or updated group identifier. |

Command errors: invalid_request, internal_error

## 3.5  delete_output_group

Delete one AC/DC Dimmer output group and all of its slave records, then publish the revised local group-mask table to the MCU service.

Implemented by: AC/DC Dimmer    Operation: Configure

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| group_id | integer | Yes | Existing positive ID | Output group to delete. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| page | string | Always | output_groups. |
| device_family | string | Always | dimmer. |
| output_count | integer | Always | Number of configured local Dimmer outputs. |
| outputs | object[] | Always | Refreshed local output list. |
| groups | object[] | Always | Refreshed saved output groups. |
| deleted_group_id | integer | Always | Deleted group identifier. |

Command errors: invalid_request, internal_error

## 3.6  rewrite_output_groups

Rewrite the complete local output-group mask table to the AC/DC Dimmer MCU. The Control API keeps the request open and returns success only after the MCU acknowledges the table.

Implemented by: AC/DC Dimmer    Operation: Configure

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| output_group_hardware_write_complete | boolean | Always | True after the MCU acknowledges the complete table. |

Command errors: internal_error

## 3.7  get_energy_state

Read the current PDU Energy Meter electrical measurements without converting missing readings to zero.

Implemented by: PDU Energy Meter    Operation: Read

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| energy | object | Always | Energy measurement object. |
| energy.voltage_v | number or null | Always | Voltage reading. |
| energy.current_a | number or null | Always | Current reading. |
| energy.active_power_w | number or null | Always | Active power. |
| energy.apparent_power_va | number or null | Always | Apparent power. |
| energy.power_factor | number or null | Always | Power factor. |
| energy.energy_kwh | number or null | Always | Accumulated energy. |
| energy.available | boolean | Always | True when every required reading is available. |

Command errors: internal_error

## 3.8  get_energy_history

Read down-sampled historical Energy Meter points for one measurement and date range.

Implemented by: PDU Energy Meter    Operation: Read

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| parameter | integer | Yes | 0 voltage, 1 current, 2 power, 3 PF, 4 VA, 5 energy | Measurement selector. |
| start_date | date | Yes | YYYY-MM-DD | Inclusive start date. |
| end_date | date | Yes | YYYY-MM-DD; maximum 365 days | Inclusive end date. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| energy_history | boolean | Always | True for an energy-history result. |
| parameter | integer | Always | Echoed measurement selector. |
| unit | string | Always | Measurement unit. |
| start_date | date | Always | Echoed start date. |
| end_date | date | Always | Echoed end date. |
| points | object[] | Always | Timestamp/value samples, limited by down-sampling. |

Command errors: invalid_request, internal_error

# 4. Configuration pages

## 4.1  get_page_configuration

Read a supported UI/configuration page using the device's existing page schema.

Implemented by: All active profiles    Operation: Read

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| page | string | Yes | Profile-specific page name | Page to read. |
| log_page | integer | No | System page; default 1 | One-based system-log page. |
| log_page_size | integer | No | System page; profile limits | Number of log rows per page. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| page | object | Always | Page-specific sections, fields, columns, rows and optional pagination. |

Command errors: invalid_request, internal_error

## 4.2  set_page_configuration

Save one supported section of a device configuration page.

Implemented by: All active profiles    Operation: Configure or admin

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| page | string | Yes | Profile-specific page name | Page containing the section. |
| section | string | Yes | Supported section name | Section to update. |
| values | object | Yes | Section-specific fields | Values to validate and persist. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| page | object | Always | Updated page or section result. |

Command errors: invalid_request, internal_error

## 4.3  mutate_page_row

Add, update or delete one row in a supported schedule, automation or security table.

Implemented by: All active profiles    Operation: Configure or admin

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| page | string | Yes | Profile-specific page name | Page containing the row collection. |
| section | string | Yes | Supported row section | Row collection to mutate. |
| operation | string | Yes | add, update or delete | Requested row operation. |
| id | integer | Update/delete | Database row ID | Existing row identifier. |
| values | object | Add/update | Section-specific fields | Row values to validate and persist. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| page | object | Always | Updated page/row collection after the mutation. |

Command errors: invalid_request, internal_error

## 4.4  execute_page_action

Execute an implemented page operation such as time synchronization, NTP test, log deletion or reboot.

Implemented by: All active profiles    Operation: Configure or admin

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| page_action | string | Yes | Profile-specific implemented action | Operation to execute. |
| datetime | string | Conditional | YYYY-MM-DD HH:MM:SS | Client/manual time for supported time actions. |
| server | string | Conditional | Host name or IP | NTP server to test. |
| log_page_size | integer | No | delete_system_logs | Requested size of the refreshed system-log page. |
| action-specific fields | mixed | Conditional | Dimmer sequence/scenario rewrites | Additional values required by the selected page action. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| result | object | Always | Action acknowledgment, test result or refreshed page data. |

Command errors: invalid_request, internal_error

# 5. Backup restore and transfers

## 5.1  transfer_upload_begin

Create or extend a chunked upload session for settings, credentials or firmware files supported by the profile.

Implemented by: All active profiles    Operation: Admin

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| kind | string | Yes | Profile-supported transfer kind | Upload/commit workflow. |
| role | string | Yes | Kind-specific file role | Role of this file in the transfer. |
| filename | string | Yes | Safe base filename | Original file name. |
| size | integer | Yes | Profile size limit | Expected file size in bytes. |
| transfer_id | string | No | Existing multi-file transfer | Attach another role to an existing transfer. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| transfer_id | string | Always | Upload-session identifier. |
| role | string | Always | Accepted file role. |
| size | integer | Always | Expected total file size. |
| received | integer | Always | Bytes currently received. |
| chunk_size | integer | Always | Maximum/recommended chunk size. |

Command errors: invalid_request, internal_error

## 5.2  transfer_upload_chunk

Append one base64-encoded byte chunk at the required upload offset.

Implemented by: All active profiles    Operation: Admin

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Open upload session | Transfer identifier. |
| role | string | Yes | Role opened by begin | File role. |
| offset | integer | Yes | Exact next byte offset | Chunk byte offset. |
| data | base64 string | Yes | Within negotiated chunk size | Chunk bytes. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| received | integer | Always | Total bytes received for this role. |
| size | integer | Always | Expected total file size. |

Command errors: invalid_request, internal_error

## 5.3  transfer_upload_finish

Finish and validate one uploaded file role before commit.

Implemented by: All active profiles    Operation: Admin

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Open upload session | Transfer identifier. |
| role | string | Yes | Completed file role | Role to finish. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| complete | boolean | Always | True after size and file validation succeeds. |
| role | string | Always | Completed role. |
| digest | string | When provided | Calculated file digest. |

Command errors: invalid_request, internal_error

## 5.4  transfer_commit

Install or restore a validated upload and report whether the device should reboot.

Implemented by: All active profiles    Operation: Admin

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Completed upload session | Transfer to commit. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| message | string | Always | Commit result description. |
| reboot_now | boolean | Always | True when firmware schedules an immediate reboot. |

Command errors: invalid_request, internal_error

## 5.5  settings_backup_download_begin

Create a temporary settings-database snapshot and start a chunked download session.

Implemented by: All active profiles    Operation: Admin

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| transfer_id | string | Always | Download-session identifier. |
| size | integer | Always | Snapshot size in bytes. |
| chunk_size | integer | Always | Maximum/recommended read size. |
| digest | string | When provided | Snapshot digest. |

Command errors: internal_error

## 5.6  transfer_download_chunk

Read one base64-encoded data chunk from an open download session.

Implemented by: All active profiles    Operation: Admin

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Open download session | Transfer identifier. |
| offset | integer | Yes | Valid byte offset | First byte to read. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| data | base64 string | Always | Returned bytes. |
| offset | integer | Always | Echoed byte offset. |
| size | integer | Always | Total download size. |
| complete | boolean | Always | True when this chunk reaches the end. |

Command errors: invalid_request, internal_error

## 5.7  transfer_download_finish

Close a download session and remove its temporary snapshot.

Implemented by: All active profiles    Operation: Admin

Parameters

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Open download session | Transfer identifier. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| complete | boolean | Always | True after cleanup. |

Command errors: invalid_request, internal_error

# Shared data schemas

These compact definitions identify the fields expected in repeated result objects. A later JSON Schema/OpenAPI file can make them machine-validatable without changing the command contract.

| Type | Required/core fields |
| --- | --- |
| DeviceInfo | device_id, name, device_type, model, serial_number, firmware_version, hardware_version |
| Topology | input_count, virtual_input_count, output_count, sensor_count |
| InputState | kind, channel, name, enabled, state, mode, active_level, debounce_ms, changed_at |
| OutputState | channel, name, enabled, state, pending, requested_level?, actual_level?, configuration?, changed_at |
| OutputConfiguration | channel, name, enabled, delays/runtimes in ms, initial_state, restart_off_time_ms |
| Mapping | input, output, behavior, legacy_code; codes: 0 none, 1 on, 2 off, 3 toggle, 4 continuous_on, 5 continuous_off |
| Schedule | id, revision, name, enabled, type, timing, actions, last_run, next_run |
| Schedule timing | one_time: datetime; recurring: time, weekdays[0=Monday..6=Sunday], optional start/end dates |
| Action | action plus params; only capability-advertised control actions are permitted |
| Automation | id, revision, name, enabled, type, trigger configuration and actions |
| Scenario | id, revision, name, default_transition_ms and targets[channel,state,level] |
| Sequence | id, revision, name, steps[scenario_id or actions, hold_ms], loop_count |
| TemperatureReading | sensor_id, name, value_c, status, timestamp |
| ChannelEnergy | channel, voltage_v?, current_a?, active_power_w?, apparent_power_va?, power_factor?, energy_wh?, timestamp |
| AccessRule | id, service, kind, value, name, enabled, revision |
| Fault | fault_id, severity, code, message, active, first_seen, last_seen |

## Configuration section schemas

| Section | Fields |
| --- | --- |
| general | name, location?, description?, language?, physical_input_count (read-only), output_count (read-only) |
| account | username, password (write-only), session_timeout_s, password_change_required |
| lan | mode[dhcp¦static], address, subnet_mask, gateway, dns_servers[], control_port = legacy_port + 3 (default 5008), legacy_port default 5005 |
| wifi | enabled, mode[dhcp¦static], ssid, password (write-only), address, subnet_mask, gateway, dns_servers[] |
| time | timezone, sync_mode[manual¦ntp], ntp_server, sync_interval_s |
| temperature | enabled, low_warning_c, high_warning_c, control/hysteresis fields where supported |
| tcp_access | tcp_enabled, legacy_port, allow_filter_enabled; tcp_enabled controls legacy_port, legacy_port + 1 and Control API TCP legacy_port + 3; rules are managed separately |
| api_access | api_enabled, IP/MAC allow rules, HTTP enabled, HTTPS enabled, authentication target, rate-limit target; api_enabled controls HTTP/HTTPS command endpoints only |
| mqtt | enabled, host, port, tls, username, password(write-only), client_id, base_topic, qos |
| tls | enabled, certificate metadata, key_present, minimum_tls_version; key material is write-only via upload |
| maintenance | log_level, automatic_backup settings, retention, update channel where supported |

# Device events

Events are unsolicited messages on subscribed TCP/WebSocket sessions or SSE streams. They use protocol, event, subscription_id and data fields; they do not contain ok or result.

{"protocol":3,"event":"output_state_changed","subscription_id":"sub-7","data":{"channel":0,"previous_state":false,"state":true,"pending":false,"source":"windows-app","revision":312,"timestamp":"2026-08-31T10:20:30+00:00"}}

| Event | Expected data fields |
| --- | --- |
| output_state_changed | channel, previous_state, state, pending, source, revision, timestamp |
| output_level_changed | channel, requested_level, actual_level, transitioning, source, revision, timestamp |
| input_state_changed | kind, channel, previous_state, state, source, revision, timestamp |
| mapping_changed | input, output, behavior, legacy_code, revision, timestamp |
| temperature_changed | sensor_id, value_c, status, timestamp |
| energy_changed | summary and/or channel readings, timestamp |
| schedule_executed | schedule_id, results, success, timestamp |
| automation_executed | automation_id, trigger, results, success, timestamp |
| sequence_state_changed | sequence_id, operation_id, state, step, loop, timestamp |
| operation_progress | operation_id, kind, stage, progress_percent, message, timestamp |
| device_fault | fault_id, severity, code, message, active, timestamp |
| configuration_changed | section, revision, source, restart_required, timestamp |
| device_rebooting | reason, reboot_in_ms, timestamp |

Synchronization: Clients must treat events as incremental updates. If revisions are skipped, reconnecting clients should call get_device_state to rebuild a complete state snapshot.

# Migration and acceptance criteria

- Migrated Relay Module, AC/DC Dimmer and PDU firmware exposes its implemented protocol 2 command subset on configured base port + 3 (5008 by default).

- Each migrated firmware family exposes the same implemented commands through POST /api/v1/command on HTTP or HTTPS.

- Discovery and heartbeat identity report the calculated Control API port so clients do not hardcode 5008.

- The standard Windows paths for all five migrated families use the JSON Control API and do not send AT commands or use the base port.

- Each configured base port remains AT-only for existing third-party or service integrations.

- Each migrated screen is tested against Relay Module, AC/DC Dimmer, PDU Energy Meter, PDU 10 kW and PDU V1 capabilities as applicable.

- Every active firmware family creates tcp_enabled and api_enabled automatically during database migration, defaults both to enabled, and persists later changes.

- With tcp_enabled false, base TCP, base + 1 and base + 3 are closed after restart while discovery, heartbeat and the web UI remain active.

- With api_enabled false, HTTP/HTTPS command routes return 503 api_disabled while raw Control API TCP and the web UI remain available.

- The mobile application uses the Control API envelope and relies on hello/capability data instead of a fixed port or device-model assumptions.

- Automated tests verify TCP framing, invalid JSON, HTTP status mapping, 1 MiB size limits, allow rules, index bounds, concurrency and reconnection.

The implemented-only catalogue must be updated whenever a new action is added to firmware dispatch, hello capabilities, the Windows/mobile client path and its automated contract tests.