Soleux Control API

TCP Default + 3 and HTTP/HTTPS Command Specification

Status: Relay Module, AC/DC Dimmer and PDU transport profiles implemented; full command catalogue remains the target contract

Audience: Device firmware, Windows application and mobile application developers

Transports: TCP legacy port + 3 (5008 by default), HTTP port 80 and HTTPS port 443

Compatibility: migrated device base ports are AT-only; Windows device paths use the Control API

Implementation status: Relay Module, AC/DC Dimmer, PDU Energy Meter, High Power PDU and PDU V1 firmware expose their implemented protocol 2 command subsets on the configured legacy TCP port plus 3 and through POST /api/v1/command over HTTP or HTTPS. The broader version 3 catalogue remains the target contract; clients must use hello/capability data and must not assume every catalogued action is available on every family.

# Document purpose

This specification defines one transport-neutral command model for Soleux device firmware, the Windows application, the upcoming mobile application and third-party HTTP/HTTPS integrations. Migrated device control no longer uses app-only J: messages. The configured base port remains available for legacy AT clients, while modern clients use the separate Control API transport.

## Normative rules

- All physical input, virtual-input and output indexes are zero-based on every transport.

- JSON booleans are true/false; API clients must not send 0/1 as boolean substitutes.

- Time durations use integer milliseconds unless a field name explicitly ends in _s.

- Electrical and environmental units are included in field names: _v, _a, _w, _va, _wh, _hz and _c.

- Timestamps use ISO 8601 with an offset. Event timestamps should be UTC where practical.

- Unknown fields may be ignored only when the negotiated protocol version permits forward-compatible extensions.

- Secrets are write-only unless a command explicitly documents that they can be returned.

- Every state-changing command identifies its required permission and produces an auditable source/session identity.

## Command groups

- Session and protocol: 7 commands. Commands used to establish a session, discover functionality and manage message delivery.

- Device information and health: 10 commands. Read device identity, full state, health, temperature and time, and perform administrative actions.

- Inputs and virtual inputs: 5 commands. Read and configure physical inputs and operate virtual inputs. All input channels are zero-based.

- Outputs: 9 commands. Live relay and dimmer control. These commands replace ON, OFF, TOGGLE, RESTART and masked AT operations.

- Input-output mappings: 5 commands. Manage mappings using named behaviors while retaining the current numeric codes for migration.

- Dimmer control: 9 commands. Device-specific brightness commands. Common output commands remain valid for on/off control.

- PDU energy and override: 7 commands. Energy and protection commands are capability-gated because meter hardware differs by PDU model.

- Schedules and automation: 12 commands. CRUD operations use stable resource objects rather than Windows-page row mutations.

- Dimmer scenarios and sequences: 11 commands. Scenario and sequence resources replace page-specific scenario/sequence mutations.

- Configuration, network and security: 9 commands. Configuration is organized into stable sections but is not coupled to a UI page layout.

- Backup, restore and firmware transfer: 7 commands. Chunked transfer commands support TCP and HTTP consistently. HTTP may additionally use direct upload endpoints later.

Catalogue size: This draft defines 91 request actions and 13 device event types.

# Transport and message envelope

The action and params object is identical on TCP, HTTP and HTTPS. Each HTTP request is dispatched through the same command handler used by the newline-delimited TCP service, so command parameters and result envelopes do not change between transports.

## Request envelope

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| protocol | integer | Yes | 2 for current Relay; 3 for target contract | Protocol version. |
| id | integer¦string | Yes | Unique among pending requests | Caller-generated correlation ID. |
| action | string | Yes | Exact action name | Command to execute. |
| params | object | No | Default {} | Command-specific parameters. |
| timeout_ms | integer | No | 100-120000 | Optional caller timeout hint; does not override safety limits. |

{"protocol":2,"id":42,"action":"set_output_state","params":{"channel":0,"state":true}}

## Success response

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| protocol | integer | Always | Protocol version. |
| id | integer¦string | Always | Original request ID. |
| ok | boolean | Always | True for a successful command. |
| result | object | Always | Command-specific expected result. |

{"protocol":2,"id":42,"ok":true,"result":{"channel":0,"requested_state":true,"actual_state":true,"pending":false}}

## Error response

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| protocol | integer | Always | Protocol version. |
| id | integer¦string¦null | Always | Original ID, or null when it could not be parsed. |
| ok | boolean | Always | False for an error. |
| error.code | string | Always | Stable machine-readable error code. |
| error.message | string | Always | Safe human-readable explanation. |
| error.details | object | Optional | Structured validation or recovery data. |
| error.retryable | boolean | Always | Whether retry may succeed without changing the request. |

{"protocol":2,"id":42,"ok":false,"error":{"code":"invalid_request","message":"output channel out of range"}}

## Transport mapping

| Transport | Request framing | Response framing | Events |
| --- | --- | --- | --- |
| TCP: legacy port + 3 | One UTF-8 JSON object per line (CRLF or LF). Default 5005 becomes Control API 5008. | One JSON response line using the common envelope. | Command responses only in the current Relay implementation. |
| HTTP port 80 | POST /api/v1/command with application/json. | HTTP status plus the common JSON envelope. | No Control API SSE endpoint is implemented yet. |
| HTTPS port 443 | Same endpoint over TLS. When SSL is enabled, HTTP receives a 307 redirect that preserves POST and body. | Same common JSON envelope. | No secure event endpoint is implemented yet. |
| WebSocket / SSE | Reserved for a future event transport. | Not currently a Control API command transport. | Planned; capability-dependent. |

## Implemented HTTP/HTTPS endpoint

Each migrated device family exposes one transport-neutral command endpoint. Every action available on that device's JSON TCP Control API is accepted at POST /api/v1/command, including state/configuration operations, page configuration and actions, system-log pagination and deletion, and supported file-transfer operations.

| Property | Implemented behavior |
| --- | --- |
| Endpoint | POST /api/v1/command |
| Media type | Request and response use application/json; the response also sets Cache-Control: no-store. |
| Request body | One JSON object using the common protocol, id, action and params fields. |
| Maximum body | 1,048,576 bytes. The limit is enforced with or without a Content-Length header. |
| Dispatch | The request is passed to the same process_tcp_json dispatcher used by the TCP Control API. |
| Client access | Existing HTTP API IP/MAC rules apply. The direct socket peer address is used; forwarding headers are not trusted for authorization. |
| HTTP/HTTPS | The same Flask endpoint is served on HTTP 80 when SSL is off and HTTPS 443 when SSL is on. |
| Legacy API | The existing /cmdapi endpoint is unchanged and remains separate from this JSON command endpoint. |

## HTTP status mapping

| Status | Meaning | Response |
| --- | --- | --- |
| 200 | The command completed successfully. | ok is true and result contains the command output. |
| 400 | Malformed body, invalid parameters or unknown action. | ok is false with invalid_request, unknown_action or another command error. |
| 403 | The peer is not allowed by the configured API IP/MAC rules. | ok is false with unauthorized. |
| 413 | The request body exceeds 1 MiB. | ok is false with message_too_large. |
| 500 | The dispatcher failed internally or produced an invalid response. | ok is false with internal_error. |

## HTTP request example

POST /api/v1/command HTTP/1.1
Content-Type: application/json

{"protocol":2,"id":"mobile-1","action":"hello","params":{}}

{"protocol":2,"id":"mobile-1","ok":true,"result":{"device":"dimmer","api_port":5008}}

## Verification commands

Run the source-level contract tests from the Soleux App project root on a developer computer with Python 3. Each active device family has its own tests folder; the Dimmer command contract is verified with:

python -m unittest discover -s "Device Firmware\Dimmer\tests" -p "test_*.py" -v

After deploying the matching firmware files and restarting WebService, run this on the device to verify the live HTTP endpoint:

curl -sS -X POST http://127.0.0.1/api/v1/command \
  -H 'Content-Type: application/json' \
  -d '{"protocol":2,"id":"test-1","action":"hello","params":{}}'

Use https://127.0.0.1/api/v1/command when SSL is enabled. A successful response has ok: true and reports the calculated api_port.

Compatibility rule: The J: prefix is not accepted on the Control API or migrated base ports. Standard Windows paths use unprefixed JSON on base port + 3 and do not send AT commands or fall back to the AT-only base port. Adding the HTTP/HTTPS endpoint does not change existing legacy AT behavior.

# Permissions and security

| Permission | Allows |
| --- | --- |
| Public | hello, ping and authenticate only. |
| Read | Read identity, state, configuration summaries, telemetry and events. |
| Control | Change live input/output/dimmer state and execute schedules/scenarios. |
| Configure | Change mappings, channel configuration, schedules, automations and non-secret settings. |
| Admin | Accounts, access rules, network/security settings, backup/restore, firmware, reboot and destructive actions. |

The implemented HTTP/HTTPS command endpoint applies each device's existing API IP/MAC allow rules to the direct socket peer. If API filtering is disabled in device settings, requests are allowed from any reachable address. Deployments should restrict HTTP to trusted local networks and use HTTPS for credentials, backup/restore, firmware or remote/mobile access. Tokens and secrets must never be written to normal logs.

## Implemented AC/DC Dimmer profile

The active Dimmer firmware accepts unprefixed protocol 2 JSON on base_port + 3 (normally 5008) and the identical command object through POST /api/v1/command over HTTP or HTTPS. The base port remains AT-only and rejects J-prefixed traffic.

All Dimmer input, output and mapping indices are zero-based. set_dimmer_level requires channel and an integer value from 0 through 100. set_output_state and restart_output use the same zero-based channel rule.

get_device_state returns requested set_pwm and live actual_pwm for every output, plus I/O state, PWM frequency, sensors, system and network data. Clients should poll this snapshot when a state-changing result reports pending: true.

The Dimmer also implements device identity, relay configuration, input/output configuration, mappings, hardware write, settings/security/system/schedule/automation/scenario/sequence pages, chunked backup/restore, paginated system logs and execute_page_action delete_system_logs.

Discovery advertises PORT, API_PORT, API_VER and CAPS. Heartbeat uses base_port + 2 and reports tcp_port, api_port and api_version. Both offsets are calculated from the configured base port.

# Common error catalogue

Every error uses the common error envelope. Commands may document additional codes.

| Code | Meaning |
| --- | --- |
| invalid_request | Envelope or JSON structure is invalid. |
| unknown_action | Action name is not recognized. |
| unsupported_command | Device recognizes the action but does not support it. |
| unsupported_protocol | No mutually supported protocol version exists. |
| authentication_required | A valid authenticated session is required. |
| authentication_failed | Credentials or token were rejected. |
| permission_denied | Session lacks the required permission. |
| invalid_parameter | A parameter has the wrong type, range or value. |
| invalid_channel | Input/output channel is outside the zero-based device range. |
| not_found | Requested resource or operation does not exist. |
| revision_conflict | expected_revision does not match current resource revision. |
| busy | A conflicting operation is active. |
| interlock | A safety or control interlock prevented the operation. |
| timeout | The operation did not complete within its allowed time. |
| rate_limited | Caller exceeded the configured request limit. |
| payload_too_large | Message or transfer exceeds advertised limits. |
| internal_error | Unexpected device-side failure; message must not expose secrets. |

# Command catalogue

Each action below defines action-specific params and the result object returned inside the common response envelope. An absent optional parameter means the device uses its current value or documented default; null is not interchangeable with omission unless explicitly stated.

# 1. Session and protocol

Commands used to establish a session, discover functionality and manage message delivery.

## 1.1  hello

Purpose: Negotiate the protocol and identify the client and device.  Support: All devices  Permission: Public

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| client_name | string | No | 1-64 characters | Name of the calling application. |
| client_version | string | No | Semantic version recommended | Calling application version. |
| protocol_min | integer | No | Default 3 | Oldest protocol version accepted by the client. |
| protocol_max | integer | No | Default 3 | Newest protocol version accepted by the client. |
| features | string[] | No | Unique values | Optional client features such as events, batch and compression. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| protocol | integer | Always | Negotiated protocol version. |
| session_id | string | Always | Identifier for this connection or HTTP session. |
| device | DeviceInfo | Always | Device identity, model and firmware summary. |
| topology | Topology | Always | Input, virtual-input and output counts. |
| limits | ProtocolLimits | Always | Payload, batch and subscription limits. |
| authentication_required | boolean | Always | Whether protected commands require authentication. |

Command-specific errors: unsupported_protocol

## 1.2  ping

Purpose: Check reachability and estimate round-trip time.  Support: All devices  Permission: Public

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| echo | any JSON value | No | Maximum 256 encoded bytes | Value returned unchanged. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| echo | any JSON value | When supplied | Original echo value. |
| server_time | datetime | Always | Current device time in ISO 8601 format. |
| uptime_ms | integer | Always | Milliseconds since the control service started. |

## 1.3  authenticate

Purpose: Create an authenticated API session.  Support: All devices with API security enabled  Permission: Public

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| method | enum | Yes | password ¦ token | Authentication method. |
| username | string | For password | 1-64 characters | Account name. |
| password | string | For password | HTTPS or protected TCP only | Account password; never logged. |
| token | string | For token | Opaque token | Previously issued API token. |
| client_name | string | No | 1-64 characters | Friendly client identifier for audit logs. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| access_token | string | Always | Bearer/session token. |
| expires_in_s | integer | Always | Token lifetime in seconds; 0 means connection lifetime. |
| permissions | string[] | Always | Granted permission names. |
| user | object | Always | Authenticated account summary without password data. |

Command-specific errors: authentication_failed, insecure_transport, account_locked

## 1.4  get_capabilities

Purpose: Return supported commands, events and device-specific limits.  Support: All devices  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| include_schemas | boolean | No | Default false | Include compact parameter/result schema metadata. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| commands | Capability[] | Always | Supported actions and required permissions. |
| events | string[] | Always | Supported event names. |
| transports | object | Always | TCP, HTTP, HTTPS and event transport availability. |
| features | string[] | Always | Device features such as dimmer, energy or Wi-Fi. |
| limits | ProtocolLimits | Always | Current service limits. |

## 1.5  subscribe

Purpose: Subscribe a persistent TCP or WebSocket session to device events.  Support: All devices  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| events | string[] | Yes | At least one supported event or * | Event names to receive. |
| channels | integer[] | No | Zero-based and unique | Optional input/output channel filter. |
| min_interval_ms | integer | No | 0-60000; default 0 | Minimum interval for coalescible telemetry events. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| subscription_id | string | Always | Identifier used by unsubscribe. |
| events | string[] | Always | Accepted event names. |
| channels | integer[] ¦ null | Always | Applied channel filter or null for all. |
| min_interval_ms | integer | Always | Applied telemetry interval. |

Command-specific errors: unsupported_event, invalid_channel, subscription_limit

## 1.6  unsubscribe

Purpose: Remove a previously created event subscription.  Support: All devices  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| subscription_id | string | Yes | Existing subscription | Subscription to remove. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| removed | boolean | Always | True when the subscription existed and was removed. |

Command-specific errors: not_found

## 1.7  batch_execute

Purpose: Execute multiple independent commands in one request.  Support: All devices  Permission: Highest permission required by a child command

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| commands | CommandRequest[] | Yes | 1 to device batch limit | Child requests without protocol/session fields. |
| stop_on_error | boolean | No | Default false | Do not start later commands after a failure. |
| atomic | boolean | No | Default false; capability dependent | Request transactional execution when supported. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| results | BatchResult[] | Always | Ordered child success/error results. |
| completed | integer | Always | Number of child commands attempted. |
| rolled_back | boolean | Always | Whether an atomic batch was rolled back. |

Command-specific errors: batch_too_large, atomic_not_supported

# 2. Device information and health

Read device identity, full state, health, temperature and time, and perform administrative actions.

## 2.1  get_device_info

Purpose: Read stable device identity and software versions.  Support: All devices  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | Command has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| device_id | string | Always | Stable unique identifier. |
| name | string | Always | Configured device name. |
| device_type | enum | Always | relay_module ¦ dimmer ¦ pdu_10kw ¦ pdu_v1 |
| model | string | Always | Hardware model. |
| serial_number | string ¦ null | Always | Serial number if available. |
| firmware_version | string | Always | Firmware/application version. |
| hardware_version | string ¦ null | Always | Hardware revision when available. |
| mac_addresses | object | Always | Available Ethernet and Wi-Fi MAC addresses. |

## 2.2  get_device_health

Purpose: Read operational health and active faults.  Support: All devices  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | Command has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| status | enum | Always | ok ¦ warning ¦ fault |
| uptime_ms | integer | Always | Service uptime. |
| faults | Fault[] | Always | Active fault records. |
| temperature_c | number ¦ null | Always | Primary device temperature. |
| cpu_percent | number ¦ null | Always | CPU usage if available. |
| memory | object ¦ null | Always | Memory totals and usage. |
| storage | object ¦ null | Always | Storage totals and usage. |
| database_ok | boolean | Always | Configuration database health. |

## 2.3  get_device_state

Purpose: Obtain the complete synchronization snapshot used after connection.  Support: All devices  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| include | string[] | No | Default inputs,outputs,sensors | Optional sections: inputs, outputs, sensors, energy, schedules, faults. |
| include_configuration | boolean | No | Default false | Include channel names and control configuration. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| revision | integer | Always | Monotonic state revision. |
| captured_at | datetime | Always | Snapshot timestamp. |
| inputs | InputState[] | When requested | Current physical and virtual input state. |
| outputs | OutputState[] | When requested | Current relay/dimmer output state. |
| sensors | SensorReading[] | When requested | Current temperature and sensor readings. |
| energy | EnergySummary ¦ null | When requested | Energy data for capable PDUs. |
| faults | Fault[] | When requested | Active faults. |

## 2.4  get_system_data

Purpose: Read diagnostic system and service data.  Support: All devices  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| include_processes | boolean | No | Default false | Include service process status when supported. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| os | object | Always | Operating-system identity and kernel. |
| services | ServiceStatus[] | Always | Core service status. |
| resources | object | Always | CPU, memory and storage values. |
| network | object | Always | Interface/link summary without credentials. |
| last_restart_reason | string ¦ null | Always | Most recent recorded restart reason. |

## 2.5  get_temperature

Purpose: Read all or one temperature sensor.  Support: All devices with temperature support  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| sensor_id | string | No | Known sensor identifier | Omit to return every temperature sensor. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| sensors | TemperatureReading[] | Always | Sensor ID, name, Celsius value, status and timestamp. |

Command-specific errors: not_found, sensor_unavailable

## 2.6  get_time

Purpose: Read device time and synchronization settings.  Support: All devices  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | Command has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| datetime | datetime | Always | Current local ISO 8601 date/time with offset. |
| timezone | string | Always | IANA time-zone name when available. |
| utc_offset_min | integer | Always | Current UTC offset in minutes. |
| sync_mode | enum | Always | manual ¦ ntp |
| ntp_server | string ¦ null | Always | Configured NTP server. |
| last_sync | datetime ¦ null | Always | Last successful synchronization time. |

## 2.7  set_time

Purpose: Set device time manually.  Support: All devices  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| datetime | datetime | Yes | ISO 8601 with offset | New date/time. |
| timezone | string | No | Supported IANA zone | New time zone. |
| write_rtc | boolean | No | Default true | Also write the hardware real-time clock. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| datetime | datetime | Always | Applied device time. |
| timezone | string | Always | Applied time zone. |
| rtc_written | boolean | Always | Whether the RTC update succeeded. |

Command-specific errors: invalid_datetime, unsupported_timezone, rtc_error

## 2.8  sync_time

Purpose: Synchronize from NTP or a supplied client time.  Support: All devices  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| source | enum | Yes | ntp ¦ client | Synchronization source. |
| datetime | datetime | For client | ISO 8601 with offset | Calling-client time. |
| server | string | No | Host name or IP | One-time NTP server override. |
| timeout_ms | integer | No | 1000-30000; default 5000 | NTP timeout. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| synchronized | boolean | Always | Whether synchronization succeeded. |
| datetime | datetime | Always | Time after synchronization. |
| offset_ms | integer ¦ null | Always | Measured NTP offset when available. |

Command-specific errors: time_sync_failed, invalid_datetime

## 2.9  test_ntp_server

Purpose: Test an NTP server without changing saved settings.  Support: All devices  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| server | string | Yes | Valid host name or IP | NTP server to test. |
| timeout_ms | integer | No | 1000-30000; default 5000 | Test timeout. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| passed | boolean | Always | Whether a valid NTP response was received. |
| latency_ms | integer ¦ null | Always | Round-trip latency. |
| offset_ms | integer ¦ null | Always | Reported clock offset. |
| message | string | Always | Human-readable outcome. |

## 2.10  reboot_device

Purpose: Restart the device after acknowledging the request.  Support: All devices  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| delay_ms | integer | No | 500-60000; default 1500 | Delay before reboot so the response can be sent. |
| reason | string | No | Maximum 128 characters | Audit-log reason. |
| force | boolean | No | Default false | Proceed despite non-critical active operations. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| accepted | boolean | Always | True when reboot was scheduled. |
| reboot_in_ms | integer | Always | Scheduled delay. |
| message | string | Always | Status message. |

Command-specific errors: busy, reboot_not_permitted

# 3. Inputs and virtual inputs

Read and configure physical inputs and operate virtual inputs. All input channels are zero-based.

## 3.1  get_inputs

Purpose: Read all physical and virtual inputs.  Support: Relay Module, Dimmer, PDU  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| include_configuration | boolean | No | Default true | Include names, enabled flags and modes. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| inputs | InputState[] | Always | Ordered physical input records. |
| virtual_inputs | InputState[] | Always | Ordered virtual-input records. |
| revision | integer | Always | State revision for synchronization. |

## 3.2  get_input

Purpose: Read one physical or virtual input.  Support: Relay Module, Dimmer, PDU  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based | Input channel. |
| kind | enum | No | physical ¦ virtual; default physical | Input collection. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| input | InputState | Always | State and configuration for the requested input. |

Command-specific errors: invalid_channel

## 3.3  set_input_configuration

Purpose: Change input name and behavior.  Support: Relay Module, Dimmer, PDU  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based physical input | Input channel. |
| name | string | No | 0-64 characters | Display name. |
| enabled | boolean | No | - | Whether the input participates in control. |
| mode | enum | No | momentary ¦ maintained ¦ pulse | Electrical/control behavior when supported. |
| active_level | enum | No | high ¦ low | Active electrical level when configurable. |
| debounce_ms | integer | No | 0-5000 | Input debounce time. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| input | InputConfiguration | Always | Complete saved configuration. |
| hardware_apply_required | boolean | Always | Whether apply_hardware_configuration must be called. |

Command-specific errors: invalid_channel, invalid_configuration

## 3.4  set_virtual_input_state

Purpose: Set and retain the state of a virtual input.  Support: Devices with virtual inputs  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based virtual input | Virtual input channel. |
| state | boolean | Yes | - | Requested logical state. |
| source | string | No | Maximum 64 characters | Audit/event source label. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Virtual input channel. |
| state | boolean | Always | Applied state. |
| changed | boolean | Always | Whether state changed. |

Command-specific errors: invalid_channel, input_disabled

## 3.5  trigger_virtual_input

Purpose: Pulse a virtual input and automatically release it.  Support: Devices with virtual inputs  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based virtual input | Virtual input channel. |
| duration_ms | integer | No | 20-60000; default 100 | Active pulse duration. |
| source | string | No | Maximum 64 characters | Audit/event source label. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Virtual input channel. |
| triggered | boolean | Always | Whether the trigger was accepted. |
| release_in_ms | integer | Always | Scheduled release delay. |

Command-specific errors: invalid_channel, input_disabled, busy

# 4. Outputs

Live relay and dimmer control. These commands replace ON, OFF, TOGGLE, RESTART and masked AT operations.

## 4.1  get_outputs

Purpose: Read all output states and optional configuration.  Support: Relay Module, Dimmer, PDU  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| include_configuration | boolean | No | Default true | Include delay, runtime and startup settings. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| outputs | OutputState[] | Always | Ordered output records. |
| revision | integer | Always | State revision. |

## 4.2  get_output

Purpose: Read one output.  Support: Relay Module, Dimmer, PDU  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based | Output channel. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| output | OutputState | Always | State and configuration for the output. |

Command-specific errors: invalid_channel

## 4.3  set_output_state

Purpose: Turn one output on or off.  Support: Relay Module, Dimmer, PDU  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based | Output channel. |
| state | boolean | Yes | - | Requested logical state. |
| transition_ms | integer | No | 0 or device-supported range | Dimmer fade duration; ignored only when capability explicitly permits. |
| source | string | No | Maximum 64 characters | Audit/event source label. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Output channel. |
| requested_state | boolean | Always | Requested state. |
| actual_state | boolean | Always | State when response was produced. |
| pending | boolean | Always | True while delays or transitions are active. |
| revision | integer | Always | New state revision. |

Command-specific errors: invalid_channel, output_disabled, interlock, busy

## 4.4  toggle_output

Purpose: Invert one output state.  Support: Relay Module, Dimmer, PDU  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based | Output channel. |
| transition_ms | integer | No | 0 or device-supported range | Dimmer fade duration. |
| source | string | No | Maximum 64 characters | Audit/event source label. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Output channel. |
| actual_state | boolean | Always | Resulting state. |
| pending | boolean | Always | Whether completion is delayed. |
| revision | integer | Always | New state revision. |

Command-specific errors: invalid_channel, output_disabled, interlock, busy

## 4.5  restart_output

Purpose: Cycle one output off and back on.  Support: Relay Module and PDU outputs; capability dependent  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based | Output channel. |
| off_time_ms | integer | No | 100-3600000; device default | Time held off. |
| restore_mode | enum | No | on ¦ previous; default on | Final state after the cycle. |
| source | string | No | Maximum 64 characters | Audit/event source label. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Output channel. |
| accepted | boolean | Always | Whether the cycle was scheduled. |
| off_time_ms | integer | Always | Applied off duration. |
| operation_id | string | Always | Identifier reported by completion events. |

Command-specific errors: invalid_channel, output_disabled, busy

## 4.6  set_multiple_outputs

Purpose: Set several outputs in one deterministic operation.  Support: Relay Module, Dimmer, PDU  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| outputs | OutputTarget[] | Yes | 1 to output count; unique channels | Each item contains channel, state and optional level/transition. |
| execution | enum | No | parallel ¦ sequential; default parallel | Execution ordering. |
| interval_ms | integer | No | 0-60000; sequential only | Delay between sequential targets. |
| stop_on_error | boolean | No | Default false | Stop after the first failed target. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| results | OutputOperationResult[] | Always | Per-output ordered results. |
| accepted | integer | Always | Accepted target count. |
| failed | integer | Always | Rejected target count. |
| operation_id | string ¦ null | Always | Group operation identifier when asynchronous. |

Command-specific errors: duplicate_channel, invalid_channel, batch_too_large

## 4.7  toggle_multiple_outputs

Purpose: Toggle several outputs together.  Support: Relay Module, Dimmer, PDU  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channels | integer[] | Yes | Unique zero-based channels | Outputs to toggle. |
| execution | enum | No | parallel ¦ sequential; default parallel | Execution ordering. |
| interval_ms | integer | No | 0-60000 | Delay between sequential targets. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| results | OutputOperationResult[] | Always | Per-output resulting states. |
| operation_id | string ¦ null | Always | Group operation identifier. |

Command-specific errors: duplicate_channel, invalid_channel

## 4.8  restart_multiple_outputs

Purpose: Restart several outputs together.  Support: Relay Module and PDU outputs; capability dependent  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channels | integer[] | Yes | Unique zero-based channels | Outputs to restart. |
| off_time_ms | integer | No | 100-3600000; device default | Off duration. |
| execution | enum | No | parallel ¦ sequential; default parallel | Execution ordering. |
| interval_ms | integer | No | 0-60000 | Sequential start interval. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| results | OutputOperationResult[] | Always | Per-output acceptance results. |
| operation_id | string | Always | Group operation identifier. |

Command-specific errors: duplicate_channel, invalid_channel, busy

## 4.9  set_output_configuration

Purpose: Change output name, delays, runtimes and startup behavior.  Support: Relay Module, Dimmer, PDU  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based | Output channel. |
| name | string | No | 0-64 characters | Display name. |
| enabled | boolean | No | - | Whether control is permitted. |
| on_delay_ms | integer | No | 0-6553500 | Delay before turning on. |
| off_delay_ms | integer | No | 0-6553500 | Delay before turning off. |
| on_runtime_ms | integer | No | 0-6553500; 0 disabled | Automatic on-duration limit. |
| off_runtime_ms | integer | No | 0-6553500; 0 disabled | Automatic off-duration limit. |
| start_delay_ms | integer | No | 0-6553500 | Startup sequencing delay. |
| initial_state | enum | No | off ¦ on ¦ restore | State after device startup. |
| restart_off_time_ms | integer | No | 100-3600000 | Default restart cycle off-time. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| output | OutputConfiguration | Always | Complete saved configuration in milliseconds. |
| hardware_apply_required | boolean | Always | Whether hardware apply is required. |

Command-specific errors: invalid_channel, invalid_configuration

# 5. Input-output mappings

Manage mappings using named behaviors while retaining the current numeric codes for migration.

## 5.1  get_mappings

Purpose: Read the complete input-output mapping matrix.  Support: Relay Module, Dimmer, PDU  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| include_disabled | boolean | No | Default true | Include mappings for disabled inputs and outputs. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| inputs | integer | Always | Number of input rows. |
| outputs | integer | Always | Number of output columns. |
| mappings | Mapping[] | Always | Sparse mapping records. |
| behavior_codes | object | Always | Numeric legacy-code to behavior-name map. |

## 5.2  get_input_mapping

Purpose: Read mappings for one input.  Support: Relay Module, Dimmer, PDU  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| input | integer | Yes | Zero-based | Input channel. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| input | integer | Always | Input channel. |
| mappings | Mapping[] | Always | One record for each mapped output. |

Command-specific errors: invalid_channel

## 5.3  set_mapping

Purpose: Create or replace one input-to-output mapping.  Support: Relay Module, Dimmer, PDU  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| input | integer | Yes | Zero-based | Input channel. |
| output | integer | Yes | Zero-based | Output channel. |
| behavior | enum | Yes | none ¦ on ¦ off ¦ toggle ¦ continuous_on ¦ continuous_off | Control behavior. |
| parameters | object | No | Behavior-specific | Reserved timing parameters for future behavior types. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| mapping | Mapping | Always | Saved input, output, behavior and legacy code. |
| hardware_apply_required | boolean | Always | Whether hardware apply is required. |

Command-specific errors: invalid_channel, invalid_behavior

## 5.4  clear_mapping

Purpose: Remove one mapping by setting its behavior to none.  Support: Relay Module, Dimmer, PDU  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| input | integer | Yes | Zero-based | Input channel. |
| output | integer | Yes | Zero-based | Output channel. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| input | integer | Always | Input channel. |
| output | integer | Always | Output channel. |
| removed | boolean | Always | True when a non-empty mapping was cleared. |

Command-specific errors: invalid_channel

## 5.5  apply_hardware_configuration

Purpose: Publish saved mappings and timing configuration to hardware-control services.  Support: Relay Module, Dimmer, PDU  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| wait | boolean | No | Default true | Wait for hardware acknowledgement. |
| timeout_ms | integer | No | 1000-60000; default 10000 | Acknowledgement timeout. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| started | boolean | Always | Whether the apply operation started. |
| completed | boolean | Always | Whether all components acknowledged before response. |
| components | ComponentApplyResult[] | Always | Per-component results. |
| operation_id | string | Always | Operation identifier. |

Command-specific errors: hardware_timeout, component_failed, busy

# 6. Dimmer control

Device-specific brightness commands. Common output commands remain valid for on/off control.

## 6.1  get_dimmer_state

Purpose: Read relay state, requested level and actual level for one dimmer.  Support: Dimmer  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based | Dimmer output channel. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Output channel. |
| state | boolean | Always | Logical on/off state. |
| requested_level | number | Always | Saved target percentage, 0.0-100.0. |
| actual_level | number | Always | Current hardware percentage, 0.0-100.0. |
| transitioning | boolean | Always | Whether a fade is active. |
| revision | integer | Always | State revision. |

Command-specific errors: invalid_channel

## 6.2  get_dimmer_levels

Purpose: Read all dimmer requested and actual levels.  Support: Dimmer  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | Command has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| outputs | DimmerState[] | Always | Ordered dimmer states. |
| revision | integer | Always | State revision. |

## 6.3  set_dimmer_level

Purpose: Set one brightness level.  Support: Dimmer  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based | Dimmer output channel. |
| level | number | Yes | 0.0-100.0 percent | Target brightness. |
| transition_ms | integer | No | 0-3600000; default 0 | Fade duration. |
| turn_on | boolean | No | Default true when level > 0 | Whether a nonzero level should enable the output. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Output channel. |
| requested_level | number | Always | Accepted target level. |
| actual_level | number | Always | Level at response time. |
| transitioning | boolean | Always | Whether transition remains active. |
| operation_id | string ¦ null | Always | Transition operation identifier. |

Command-specific errors: invalid_channel, invalid_level, output_disabled

## 6.4  set_multiple_dimmer_levels

Purpose: Set several brightness levels together.  Support: Dimmer  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| outputs | DimmerTarget[] | Yes | Unique zero-based channels | Channel, level and optional transition for each output. |
| execution | enum | No | parallel ¦ sequential; default parallel | Execution ordering. |
| interval_ms | integer | No | 0-60000 | Sequential target interval. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| results | DimmerOperationResult[] | Always | Per-channel acceptance and target levels. |
| operation_id | string ¦ null | Always | Group transition identifier. |

Command-specific errors: duplicate_channel, invalid_channel, invalid_level

## 6.5  dimmer_on

Purpose: Turn on one dimmer using its saved requested level.  Support: Dimmer  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based | Dimmer channel. |
| transition_ms | integer | No | 0-3600000 | Fade-in duration. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| dimmer | DimmerState | Always | Resulting dimmer state. |

Command-specific errors: invalid_channel, output_disabled

## 6.6  dimmer_off

Purpose: Turn off one dimmer without discarding its saved level.  Support: Dimmer  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based | Dimmer channel. |
| transition_ms | integer | No | 0-3600000 | Fade-out duration. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| dimmer | DimmerState | Always | Resulting dimmer state. |

Command-specific errors: invalid_channel, output_disabled

## 6.7  toggle_dimmer

Purpose: Toggle one dimmer while retaining its target level.  Support: Dimmer  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based | Dimmer channel. |
| transition_ms | integer | No | 0-3600000 | Fade duration. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| dimmer | DimmerState | Always | Resulting dimmer state. |

Command-specific errors: invalid_channel, output_disabled

## 6.8  get_dimmer_frequency

Purpose: Read configured dimmer PWM/drive frequency.  Support: Dimmer  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | Command has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| frequency_hz | integer | Always | Configured frequency. |
| allowed_hz | integer[] ¦ range | Always | Supported values or range. |
| apply_required | boolean | Always | Whether a pending value awaits hardware apply. |

## 6.9  set_dimmer_frequency

Purpose: Change dimmer PWM/drive frequency.  Support: Dimmer  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| frequency_hz | integer | Yes | One of allowed_hz | Requested frequency. |
| apply_now | boolean | No | Default true | Apply immediately to hardware. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| frequency_hz | integer | Always | Applied or saved frequency. |
| applied | boolean | Always | Whether hardware accepted the value. |
| restart_required | boolean | Always | Whether device restart is required. |

Command-specific errors: invalid_frequency, hardware_error

# 7. PDU energy and override

Energy and protection commands are capability-gated because meter hardware differs by PDU model.

## 7.1  get_energy_summary

Purpose: Read aggregate electrical measurements and counters.  Support: Energy-capable PDU  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| include_channels | boolean | No | Default false | Include per-channel measurements. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| captured_at | datetime | Always | Measurement time. |
| voltage_v | number ¦ null | Always | Supply voltage. |
| current_a | number ¦ null | Always | Total current. |
| active_power_w | number ¦ null | Always | Active power. |
| apparent_power_va | number ¦ null | Always | Apparent power. |
| power_factor | number ¦ null | Always | Power factor, 0.0-1.0. |
| frequency_hz | number ¦ null | Always | Mains frequency. |
| energy_wh | number ¦ null | Always | Accumulated energy. |
| channels | ChannelEnergy[] | When requested | Per-channel values supported by the meter. |

## 7.2  get_channel_energy

Purpose: Read measurements for one metered output.  Support: PDU with per-channel metering  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based metered output | Output channel. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| energy | ChannelEnergy | Always | Channel voltage/current/power/energy and timestamp. |

Command-specific errors: invalid_channel, meter_unavailable

## 7.3  get_override_state

Purpose: Read manual or safety override status.  Support: PDU  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | No | Zero-based; omit for all | Optional output channel. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| overrides | OverrideState[] | Always | Active/inactive override state, reason and source. |

## 7.4  set_override_state

Purpose: Enable or clear an override where hardware supports remote override control.  Support: Capability-dependent PDU  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | Zero-based | Output channel. |
| enabled | boolean | Yes | - | Requested override state. |
| state | boolean | When enabling | - | Forced output state. |
| reason | string | Yes | 1-128 characters | Audit reason. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| override | OverrideState | Always | Applied override state. |
| output | OutputState | Always | Resulting output state. |

Command-specific errors: unsupported_command, invalid_channel, safety_interlock

## 7.5  reset_energy_counters

Purpose: Reset supported accumulated energy counters.  Support: Energy-capable PDU  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| scope | enum | Yes | all ¦ channel | Counter scope. |
| channel | integer | For channel | Zero-based | Metered output channel. |
| confirmation | string | Yes | Literal RESET | Explicit destructive-operation confirmation. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| reset | boolean | Always | Whether counters were reset. |
| reset_at | datetime | Always | Reset timestamp. |
| channels | integer[] | Always | Affected channels. |

Command-specific errors: confirmation_required, meter_unavailable

## 7.6  get_power_limits

Purpose: Read configured warning and trip limits.  Support: PDU with protection limits  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | Command has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| limits | PowerLimits | Always | Voltage, current, power and temperature limits with hysteresis. |

## 7.7  set_power_limits

Purpose: Change supported power warning/trip limits.  Support: PDU with protection limits  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| limits | PowerLimitsPatch | Yes | Only capability-advertised fields | Fields to update. |
| apply_now | boolean | No | Default true | Apply to protection service immediately. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| limits | PowerLimits | Always | Complete applied limits. |
| applied | boolean | Always | Whether protection service acknowledged. |

Command-specific errors: invalid_limit, hardware_error

# 8. Schedules and automation

CRUD operations use stable resource objects rather than Windows-page row mutations.

## 8.1  get_schedules

Purpose: List one-time and recurring schedules.  Support: All devices with scheduling  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| type | enum | No | one_time ¦ recurring | Optional schedule type filter. |
| enabled | boolean | No | - | Optional enabled-state filter. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| schedules | Schedule[] | Always | Schedule records in stable ID order. |
| timezone | string | Always | Time zone used for execution. |

## 8.2  get_schedule

Purpose: Read one schedule.  Support: All devices with scheduling  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| schedule_id | string | Yes | Existing ID | Schedule identifier. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| schedule | Schedule | Always | Complete schedule. |

Command-specific errors: not_found

## 8.3  create_schedule

Purpose: Create a one-time or recurring schedule.  Support: All devices with scheduling  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| schedule | ScheduleCreate | Yes | See Schedule schema | Name, type, timing, actions and enabled state. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| schedule | Schedule | Always | Created schedule with generated ID. |

Command-specific errors: invalid_schedule, schedule_limit, conflict

## 8.4  update_schedule

Purpose: Patch an existing schedule.  Support: All devices with scheduling  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| schedule_id | string | Yes | Existing ID | Schedule identifier. |
| changes | SchedulePatch | Yes | At least one field | Fields to change. |
| expected_revision | integer | No | Current revision | Optimistic concurrency check. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| schedule | Schedule | Always | Updated schedule. |

Command-specific errors: not_found, invalid_schedule, revision_conflict

## 8.5  delete_schedule

Purpose: Delete a schedule.  Support: All devices with scheduling  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| schedule_id | string | Yes | Existing ID | Schedule identifier. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| deleted | boolean | Always | Whether the schedule existed and was deleted. |
| schedule_id | string | Always | Requested identifier. |

Command-specific errors: not_found

## 8.6  enable_schedule

Purpose: Enable or disable a schedule without changing its definition.  Support: All devices with scheduling  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| schedule_id | string | Yes | Existing ID | Schedule identifier. |
| enabled | boolean | Yes | - | Requested state. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| schedule | Schedule | Always | Updated schedule. |

Command-specific errors: not_found

## 8.7  execute_schedule

Purpose: Execute a schedule's actions immediately for testing.  Support: All devices with scheduling  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| schedule_id | string | Yes | Existing ID | Schedule identifier. |
| dry_run | boolean | No | Default false | Validate and return planned actions without applying them. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| accepted | boolean | Always | Whether execution was accepted. |
| actions | ActionResult[] | Always | Validation or execution result for each action. |
| operation_id | string ¦ null | Always | Asynchronous operation identifier. |

Command-specific errors: not_found, action_failed, busy

## 8.8  get_automations

Purpose: List configured automations.  Support: Capability-dependent  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| type | enum | No | ping_watchdog ¦ delayed_action ¦ temperature_control ¦ tcp_trigger ¦ api_trigger | Optional type filter. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| automations | Automation[] | Always | Automation records. |

## 8.9  create_automation

Purpose: Create an automation.  Support: Capability-dependent  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| automation | AutomationCreate | Yes | See Automation schema | Type, trigger, actions, name and enabled state. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| automation | Automation | Always | Created automation with generated ID. |

Command-specific errors: invalid_automation, automation_limit

## 8.10  update_automation

Purpose: Patch an automation.  Support: Capability-dependent  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| automation_id | string | Yes | Existing ID | Automation identifier. |
| changes | AutomationPatch | Yes | At least one field | Fields to change. |
| expected_revision | integer | No | Current revision | Optimistic concurrency check. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| automation | Automation | Always | Updated automation. |

Command-specific errors: not_found, invalid_automation, revision_conflict

## 8.11  delete_automation

Purpose: Delete an automation.  Support: Capability-dependent  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| automation_id | string | Yes | Existing ID | Automation identifier. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| deleted | boolean | Always | Whether the resource was deleted. |
| automation_id | string | Always | Requested identifier. |

Command-specific errors: not_found

## 8.12  enable_automation

Purpose: Enable or disable an automation.  Support: Capability-dependent  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| automation_id | string | Yes | Existing ID | Automation identifier. |
| enabled | boolean | Yes | - | Requested state. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| automation | Automation | Always | Updated automation. |

Command-specific errors: not_found

# 9. Dimmer scenarios and sequences

Scenario and sequence resources replace page-specific scenario/sequence mutations.

## 9.1  get_scenarios

Purpose: List dimmer scenarios.  Support: Dimmer  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | Command has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| scenarios | Scenario[] | Always | Scenario definitions. |

## 9.2  create_scenario

Purpose: Create a named set of dimmer/output targets.  Support: Dimmer  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| scenario | ScenarioCreate | Yes | At least one target | Name, targets and default transition. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| scenario | Scenario | Always | Created scenario. |

Command-specific errors: invalid_scenario, scenario_limit

## 9.3  update_scenario

Purpose: Patch a scenario.  Support: Dimmer  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| scenario_id | string | Yes | Existing ID | Scenario identifier. |
| changes | ScenarioPatch | Yes | At least one field | Fields to change. |
| expected_revision | integer | No | Current revision | Optimistic concurrency check. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| scenario | Scenario | Always | Updated scenario. |

Command-specific errors: not_found, invalid_scenario, revision_conflict

## 9.4  delete_scenario

Purpose: Delete a scenario.  Support: Dimmer  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| scenario_id | string | Yes | Existing ID | Scenario identifier. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| deleted | boolean | Always | Deletion result. |
| scenario_id | string | Always | Requested identifier. |

Command-specific errors: not_found, resource_in_use

## 9.5  execute_scenario

Purpose: Apply a scenario.  Support: Dimmer  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| scenario_id | string | Yes | Existing ID | Scenario identifier. |
| transition_ms | integer | No | 0-3600000 | Override default transition. |
| dry_run | boolean | No | Default false | Validate without changing outputs. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| accepted | boolean | Always | Whether execution was accepted. |
| results | DimmerOperationResult[] | Always | Per-target results. |
| operation_id | string ¦ null | Always | Transition identifier. |

Command-specific errors: not_found, action_failed

## 9.6  get_sequences

Purpose: List scenario sequences.  Support: Dimmer  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | Command has no action-specific parameters. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| sequences | Sequence[] | Always | Sequence definitions. |

## 9.7  create_sequence

Purpose: Create an ordered scenario/action sequence.  Support: Dimmer  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| sequence | SequenceCreate | Yes | At least one step | Name, steps and loop settings. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| sequence | Sequence | Always | Created sequence. |

Command-specific errors: invalid_sequence, sequence_limit

## 9.8  update_sequence

Purpose: Patch a sequence.  Support: Dimmer  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| sequence_id | string | Yes | Existing ID | Sequence identifier. |
| changes | SequencePatch | Yes | At least one field | Fields to change. |
| expected_revision | integer | No | Current revision | Optimistic concurrency check. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| sequence | Sequence | Always | Updated sequence. |

Command-specific errors: not_found, invalid_sequence, revision_conflict

## 9.9  delete_sequence

Purpose: Delete a sequence.  Support: Dimmer  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| sequence_id | string | Yes | Existing ID | Sequence identifier. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| deleted | boolean | Always | Deletion result. |
| sequence_id | string | Always | Requested identifier. |

Command-specific errors: not_found, resource_in_use

## 9.10  start_sequence

Purpose: Start a sequence.  Support: Dimmer  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| sequence_id | string | Yes | Existing ID | Sequence identifier. |
| start_step | integer | No | Zero-based; default 0 | First step to execute. |
| loop_count | integer | No | 0-65535; 0 uses saved setting | Runtime loop override. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| accepted | boolean | Always | Whether start was accepted. |
| operation_id | string | Always | Running sequence identifier. |
| sequence_id | string | Always | Definition identifier. |
| step | integer | Always | Initial zero-based step. |

Command-specific errors: not_found, busy, invalid_step

## 9.11  stop_sequence

Purpose: Stop one running sequence operation.  Support: Dimmer  Permission: Control

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| operation_id | string | Yes | Running operation | Operation returned by start_sequence. |
| output_behavior | enum | No | hold ¦ off ¦ restore; default hold | What outputs do after stopping. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| stopped | boolean | Always | Whether a running operation was stopped. |
| operation_id | string | Always | Requested operation identifier. |
| outputs | DimmerState[] | Always | Resulting output states. |

Command-specific errors: not_found

# 10. Configuration, network and security

Configuration is organized into stable sections but is not coupled to a UI page layout.

## 10.1  get_configuration

Purpose: Read one or more configuration sections.  Support: All devices  Permission: Read

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| sections | string[] | No | Default all permitted | general, account, lan, wifi, time, temperature, tcp_access, api_access, mqtt, tls, maintenance. |
| include_secrets | boolean | No | Default false; Admin only | Return only retrievable secrets; passwords normally remain write-only. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| configuration | object | Always | Map of section name to typed section object. |
| revision | integer | Always | Configuration revision. |
| restart_required | boolean | Always | Whether pending configuration needs a restart. |

Command-specific errors: unknown_section, permission_denied

## 10.2  set_configuration

Purpose: Patch one configuration section.  Support: All devices  Permission: Configure or Admin by section

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| section | string | Yes | Supported configuration section | Section to change. |
| values | object | Yes | Section-specific schema | Fields to patch; omitted fields are unchanged. |
| expected_revision | integer | No | Current revision | Optimistic concurrency check. |
| apply_now | boolean | No | Default true | Apply service changes immediately when safe. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| section | string | Always | Updated section. |
| values | object | Always | Complete sanitized section after update. |
| revision | integer | Always | New configuration revision. |
| applied | boolean | Always | Whether change is active. |
| restart_required | boolean | Always | Whether restart is required. |

Command-specific errors: unknown_section, invalid_configuration, revision_conflict

## 10.3  scan_wifi

Purpose: Scan for nearby Wi-Fi networks.  Support: Devices with Wi-Fi  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| timeout_ms | integer | No | 2000-30000; default 10000 | Scan timeout. |
| include_hidden | boolean | No | Default false | Include hidden-network placeholders. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| networks | WifiNetwork[] | Always | SSID, signal, security and channel; never credentials. |
| duration_ms | integer | Always | Scan duration. |

Command-specific errors: wifi_unavailable, busy

## 10.4  test_network

Purpose: Test name resolution and TCP reachability without saving settings.  Support: All devices  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| host | string | Yes | Host name or IP | Target host. |
| port | integer | No | 1-65535 | Optional TCP port. |
| timeout_ms | integer | No | 500-30000; default 5000 | Test timeout. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| dns_ok | boolean ¦ null | Always | DNS result when a host name is supplied. |
| resolved_addresses | string[] | Always | Resolved addresses. |
| reachable | boolean | Always | ICMP or TCP reachability result. |
| latency_ms | integer ¦ null | Always | Measured latency. |
| message | string | Always | Outcome summary. |

## 10.5  test_mqtt

Purpose: Test MQTT connection and optional publish without saving settings.  Support: Devices with MQTT  Permission: Configure

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| host | string | Yes | Host name or IP | Broker. |
| port | integer | Yes | 1-65535 | Broker port. |
| tls | boolean | No | Default false | Use TLS. |
| username | string | No | - | Broker username. |
| password | string | No | Write-only | Broker password. |
| topic | string | No | Valid MQTT topic | Optional test publish topic. |
| timeout_ms | integer | No | 1000-30000 | Connection timeout. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| connected | boolean | Always | Connection result. |
| published | boolean ¦ null | Always | Publish result when requested. |
| latency_ms | integer ¦ null | Always | Connection latency. |
| message | string | Always | Sanitized outcome. |

Command-specific errors: tls_error, connection_failed, authentication_failed

## 10.6  get_access_rules

Purpose: List TCP or API IP/MAC allow rules.  Support: All devices  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| service | enum | Yes | legacy_tcp ¦ control_api | Rule collection. |
| kind | enum | No | ip ¦ mac | Optional rule type filter. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| service | enum | Always | Requested service. |
| rules | AccessRule[] | Always | Ordered allow rules. |
| filter_enabled | boolean | Always | Whether rules are enforced. |

## 10.7  add_access_rule

Purpose: Add an IP/CIDR or MAC allow rule.  Support: All devices  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| service | enum | Yes | legacy_tcp ¦ control_api | Target service. |
| kind | enum | Yes | ip ¦ mac | Rule type. |
| value | string | Yes | IP/CIDR or MAC syntax | Allowed value. |
| name | string | No | 0-64 characters | Friendly label. |
| enabled | boolean | No | Default true | Rule state. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| rule | AccessRule | Always | Created rule with ID. |

Command-specific errors: invalid_rule, duplicate_rule, rule_limit

## 10.8  update_access_rule

Purpose: Patch an access rule.  Support: All devices  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| rule_id | string | Yes | Existing ID | Rule identifier. |
| changes | AccessRulePatch | Yes | At least one field | value, name or enabled. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| rule | AccessRule | Always | Updated rule. |

Command-specific errors: not_found, invalid_rule, duplicate_rule

## 10.9  delete_access_rule

Purpose: Delete an access rule.  Support: All devices  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| rule_id | string | Yes | Existing ID | Rule identifier. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| deleted | boolean | Always | Deletion result. |
| rule_id | string | Always | Requested identifier. |

Command-specific errors: not_found

# 11. Backup, restore and firmware transfer

Chunked transfer commands support TCP and HTTP consistently. HTTP may additionally use direct upload endpoints later.

## 11.1  upload_begin

Purpose: Create a settings, certificate or firmware upload session.  Support: All devices; kind capability-dependent  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| kind | enum | Yes | settings_restore ¦ firmware ¦ tls_certificate ¦ tls_key | Upload purpose. |
| file_name | string | Yes | Base name only, 1-128 characters | Display/audit file name. |
| size_bytes | integer | Yes | 1 to advertised limit | Total decoded size. |
| sha256 | string | Yes | 64 lowercase hex characters | Expected file hash. |
| metadata | object | No | Kind-specific | Version or certificate metadata. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| transfer_id | string | Always | Upload session identifier. |
| chunk_size | integer | Always | Required/maximum decoded chunk bytes. |
| expires_in_s | integer | Always | Idle session lifetime. |
| next_offset | integer | Always | Initial byte offset, normally 0. |

Command-specific errors: file_too_large, invalid_hash, transfer_limit, busy

## 11.2  upload_chunk

Purpose: Append one base64-encoded upload chunk.  Support: All devices  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Active upload | Upload identifier. |
| offset | integer | Yes | Exactly next_offset | Decoded byte offset. |
| data | base64 string | Yes | At most chunk_size decoded bytes | Chunk data. |
| chunk_sha256 | string | No | 64 hex characters | Optional per-chunk integrity check. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| received_bytes | integer | Always | Total decoded bytes received. |
| next_offset | integer | Always | Required next byte offset. |
| complete | boolean | Always | Whether declared size has been received. |

Command-specific errors: not_found, offset_mismatch, invalid_base64, hash_mismatch

## 11.3  upload_finish

Purpose: Validate the uploaded file and close data transfer.  Support: All devices  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Active upload | Upload identifier. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| validated | boolean | Always | Size/hash and kind validation result. |
| sha256 | string | Always | Calculated full-file hash. |
| metadata | object | Always | Validated firmware/settings/certificate metadata. |
| commit_required | boolean | Always | Whether upload_commit must follow. |

Command-specific errors: not_found, incomplete_transfer, hash_mismatch, invalid_file

## 11.4  upload_commit

Purpose: Install or restore a validated upload.  Support: All devices; kind capability-dependent  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Validated upload | Upload identifier. |
| reboot | boolean | No | Default true when required | Allow automatic reboot. |
| confirmation | string | Yes | Kind-specific literal supplied by upload_finish | Explicit destructive-operation confirmation. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| accepted | boolean | Always | Whether commit started. |
| operation_id | string | Always | Install/restore operation identifier. |
| reboot_required | boolean | Always | Whether reboot is required. |
| reboot_in_ms | integer ¦ null | Always | Automatic reboot delay. |

Command-specific errors: not_found, confirmation_required, validation_expired, install_failed

## 11.5  download_begin

Purpose: Create a settings-backup or diagnostic download.  Support: All devices  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| kind | enum | Yes | settings_backup ¦ diagnostics | Download content. |
| options | object | No | Kind-specific | Optional inclusion/redaction settings. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| transfer_id | string | Always | Download session identifier. |
| file_name | string | Always | Suggested safe file name. |
| size_bytes | integer | Always | Total decoded size. |
| sha256 | string | Always | File hash. |
| chunk_size | integer | Always | Maximum read chunk bytes. |
| expires_in_s | integer | Always | Idle session lifetime. |

Command-specific errors: busy, generation_failed

## 11.6  download_chunk

Purpose: Read one base64-encoded download chunk.  Support: All devices  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Active download | Download identifier. |
| offset | integer | Yes | 0 to size_bytes | Decoded byte offset. |
| length | integer | No | 1 to chunk_size | Requested decoded length. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| offset | integer | Always | Returned decoded byte offset. |
| data | base64 string | Always | Chunk data. |
| length | integer | Always | Decoded chunk bytes. |
| next_offset | integer | Always | Next sequential offset. |
| complete | boolean | Always | Whether end-of-file was reached. |

Command-specific errors: not_found, invalid_offset

## 11.7  download_finish

Purpose: Close and delete a generated download session.  Support: All devices  Permission: Admin

Parameters

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Active download | Download identifier. |

Expected result

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| closed | boolean | Always | Whether session was closed. |
| transfer_id | string | Always | Requested identifier. |

Command-specific errors: not_found

# Shared data schemas

These compact definitions identify the fields expected in repeated result objects. A later JSON Schema/OpenAPI file can make them machine-validatable without changing the command contract.

| Type | Required/core fields |
| --- | --- |
| DeviceInfo | device_id, name, device_type, model, serial_number, firmware_version, hardware_version |
| Topology | input_count, virtual_input_count, output_count, sensor_count |
| InputState | kind, channel, name, enabled, state, mode, active_level, debounce_ms, changed_at |
| OutputState | channel, name, enabled, state, pending, requested_level?, actual_level?, configuration?, changed_at |
| OutputConfiguration | channel, name, enabled, delays/runtimes in ms, initial_state, restart_off_time_ms |
| Mapping | input, output, behavior, legacy_code; codes: 0 none, 1 on, 2 off, 3 toggle, 4 continuous_on, 5 continuous_off |
| Schedule | id, revision, name, enabled, type, timing, actions, last_run, next_run |
| Schedule timing | one_time: datetime; recurring: time, weekdays[0=Monday..6=Sunday], optional start/end dates |
| Action | action plus params; only capability-advertised control actions are permitted |
| Automation | id, revision, name, enabled, type, trigger configuration and actions |
| Scenario | id, revision, name, default_transition_ms and targets[channel,state,level] |
| Sequence | id, revision, name, steps[scenario_id or actions, hold_ms], loop_count |
| TemperatureReading | sensor_id, name, value_c, status, timestamp |
| ChannelEnergy | channel, voltage_v?, current_a?, active_power_w?, apparent_power_va?, power_factor?, energy_wh?, timestamp |
| AccessRule | id, service, kind, value, name, enabled, revision |
| Fault | fault_id, severity, code, message, active, first_seen, last_seen |

## Configuration section schemas

| Section | Fields |
| --- | --- |
| general | name, location?, description?, language?, physical_input_count (read-only), output_count (read-only) |
| account | username, password (write-only), session_timeout_s, password_change_required |
| lan | mode[dhcp¦static], address, subnet_mask, gateway, dns_servers[], control_port = legacy_port + 3 (default 5008), legacy_port default 5005 |
| wifi | enabled, mode[dhcp¦static], ssid, password (write-only), address, subnet_mask, gateway, dns_servers[] |
| time | timezone, sync_mode[manual¦ntp], ntp_server, sync_interval_s |
| temperature | enabled, low_warning_c, high_warning_c, control/hysteresis fields where supported |
| tcp_access | legacy_enabled, legacy_port, allow_filter_enabled; rules are managed separately |
| api_access | IP/MAC allow rules, HTTP enabled, HTTPS enabled, authentication target, rate-limit target |
| mqtt | enabled, host, port, tls, username, password(write-only), client_id, base_topic, qos |
| tls | enabled, certificate metadata, key_present, minimum_tls_version; key material is write-only via upload |
| maintenance | log_level, automatic_backup settings, retention, update channel where supported |

# Device events

Events are unsolicited messages on subscribed TCP/WebSocket sessions or SSE streams. They use protocol, event, subscription_id and data fields; they do not contain ok or result.

{"protocol":3,"event":"output_state_changed","subscription_id":"sub-7","data":{"channel":0,"previous_state":false,"state":true,"pending":false,"source":"windows-app","revision":312,"timestamp":"2026-08-31T10:20:30+00:00"}}

| Event | Expected data fields |
| --- | --- |
| output_state_changed | channel, previous_state, state, pending, source, revision, timestamp |
| output_level_changed | channel, requested_level, actual_level, transitioning, source, revision, timestamp |
| input_state_changed | kind, channel, previous_state, state, source, revision, timestamp |
| mapping_changed | input, output, behavior, legacy_code, revision, timestamp |
| temperature_changed | sensor_id, value_c, status, timestamp |
| energy_changed | summary and/or channel readings, timestamp |
| schedule_executed | schedule_id, results, success, timestamp |
| automation_executed | automation_id, trigger, results, success, timestamp |
| sequence_state_changed | sequence_id, operation_id, state, step, loop, timestamp |
| operation_progress | operation_id, kind, stage, progress_percent, message, timestamp |
| device_fault | fault_id, severity, code, message, active, timestamp |
| configuration_changed | section, revision, source, restart_required, timestamp |
| device_rebooting | reason, reboot_in_ms, timestamp |

Synchronization: Clients must treat events as incremental updates. If revisions are skipped, reconnecting clients should call get_device_state to rebuild a complete state snapshot.

# Migration and acceptance criteria

- Migrated Relay Module, AC/DC Dimmer and PDU firmware exposes its implemented protocol 2 command subset on configured base port + 3 (5008 by default).

- Each migrated firmware family exposes the same implemented commands through POST /api/v1/command on HTTP or HTTPS.

- Discovery and heartbeat identity report the calculated Control API port so clients do not hardcode 5008.

- The standard Windows paths for all five migrated families use the JSON Control API and do not send AT commands or use the base port.

- Each configured base port remains AT-only for existing third-party or service integrations.

- Each migrated screen is tested against Relay Module, AC/DC Dimmer, PDU Energy Meter, PDU 10 kW and PDU V1 capabilities as applicable.

- The mobile application uses the Control API envelope and relies on hello/capability data instead of a fixed port or device-model assumptions.

- Automated tests verify TCP framing, invalid JSON, HTTP status mapping, 1 MiB size limits, allow rules, index bounds, concurrency and reconnection.

Next protocol milestone: Promote the implemented device-family protocol 2 subsets toward the version 3 target only after command names, authentication, permissions, event transport, output delay units, schedule semantics and cross-device capabilities are confirmed.