Soleux Control API

TCP Default + 3 and HTTP/HTTPS Command Specification

Status: implemented protocol 2 command catalogue for all five active device profiles

Audience: Device firmware, Windows application and mobile application developers

Transports: TCP legacy port + 3 (5008 by default), HTTP port 80 and HTTPS port 443

Compatibility: migrated device base ports are AT-only; Windows device paths use the Control API

Implementation status: Relay Module, AC/DC Dimmer, PDU Energy Meter, High Power PDU and PDU V1 firmware expose the implemented protocol 2 command subsets on the configured base TCP port plus 3 and through POST /api/v1/command over HTTP or HTTPS. This specification lists only those implemented actions; device-specific actions remain capability-gated.

# Document purpose

This specification defines the implemented transport-neutral command model for Soleux device firmware, the Windows application and mobile clients. Migrated device control uses the dedicated Control API transport; the configured base port remains available for legacy AT clients.

## Normative rules

- All physical input, virtual-input and output indexes are zero-based on every transport.

- JSON booleans are true/false; API clients must not send 0/1 as boolean substitutes.

- Time durations use integer milliseconds unless a field name explicitly ends in _s.

- Electrical and environmental units are included in field names: _v, _a, _w, _va, _wh, _hz and _c.

- Timestamps use ISO 8601 with an offset. Event timestamps should be UTC where practical.

- Unknown fields may be ignored only when the negotiated protocol version permits forward-compatible extensions.

- Secrets are write-only unless a command explicitly documents that they can be returned.

- Every state-changing command identifies its required permission and produces an auditable source/session identity.

## Command groups

- Session and identity: 4 implemented actions.

- Core device state: 4 implemented actions.

- Input control: 3 implemented actions on supported profiles.

- Output control: 3 implemented actions on all profiles.

- Mapping and hardware control: 2 implemented actions on supported profiles.

- Dimmer control: 2 implemented actions on the AC/DC Dimmer.

- Energy metering: 2 implemented actions on the PDU Energy Meter.

- Configuration pages: 4 implemented actions on all profiles.

- Transfers: 7 implemented actions on all profiles.

Catalogue size: 27 implemented request actions.

# Transport and message envelope

The action and params object is identical on TCP, HTTP and HTTPS. Each HTTP request is dispatched through the same command handler used by the newline-delimited TCP service, so command parameters and result envelopes do not change between transports.

## Request envelope

| Field | Type | Required | Constraints/default | Description |
| --- | --- | --- | --- | --- |
| protocol | integer | Yes | 2 for current Relay; 3 for target contract | Protocol version. |
| id | integer¦string | Yes | Unique among pending requests | Caller-generated correlation ID. |
| action | string | Yes | Exact action name | Command to execute. |
| params | object | No | Default {} | Command-specific parameters. |
| timeout_ms | integer | No | 100-120000 | Optional caller timeout hint; does not override safety limits. |

{"protocol":2,"id":42,"action":"set_output_state","params":{"channel":0,"state":true}}

## Success response

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| protocol | integer | Always | Protocol version. |
| id | integer¦string | Always | Original request ID. |
| ok | boolean | Always | True for a successful command. |
| result | object | Always | Command-specific expected result. |

{"protocol":2,"id":42,"ok":true,"result":{"channel":0,"requested_state":true,"actual_state":true,"pending":false}}

## Error response

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| protocol | integer | Always | Protocol version. |
| id | integer¦string¦null | Always | Original ID, or null when it could not be parsed. |
| ok | boolean | Always | False for an error. |
| error.code | string | Always | Stable machine-readable error code. |
| error.message | string | Always | Safe human-readable explanation. |
| error.details | object | Optional | Structured validation or recovery data. |
| error.retryable | boolean | Always | Whether retry may succeed without changing the request. |

{"protocol":2,"id":42,"ok":false,"error":{"code":"invalid_request","message":"output channel out of range"}}

## Transport mapping

| Transport | Request framing | Response framing | Events |
| --- | --- | --- | --- |
| TCP: legacy port + 3 | One UTF-8 JSON object per line (CRLF or LF). Default 5005 becomes Control API 5008. | One JSON response line using the common envelope. | Command responses only in the current Relay implementation. |
| HTTP port 80 | POST /api/v1/command with application/json. | HTTP status plus the common JSON envelope. | No Control API SSE endpoint is implemented yet. |
| HTTPS port 443 | Same endpoint over TLS. When SSL is enabled, HTTP receives a 307 redirect that preserves POST and body. | Same common JSON envelope. | No secure event endpoint is implemented yet. |
| WebSocket / SSE | Reserved for a future event transport. | Not currently a Control API command transport. | Planned; capability-dependent. |

## Implemented HTTP/HTTPS endpoint

Each migrated device family exposes one transport-neutral command endpoint. Every action available on that device's JSON TCP Control API is accepted at POST /api/v1/command, including state/configuration operations, page configuration and actions, system-log pagination and deletion, and supported file-transfer operations.

| Property | Implemented behavior |
| --- | --- |
| Endpoint | POST /api/v1/command |
| Media type | Request and response use application/json; the response also sets Cache-Control: no-store. |
| Request body | One JSON object using the common protocol, id, action and params fields. |
| Maximum body | 1,048,576 bytes. The limit is enforced with or without a Content-Length header. |
| Dispatch | The request is passed to the same process_tcp_json dispatcher used by the TCP Control API. |
| Client access | api_enabled must be true. Existing HTTP API IP/MAC rules then apply to the direct socket peer; forwarding headers are not trusted for authorization. |
| HTTP/HTTPS | The same Flask endpoint is served on HTTP 80 when SSL is off and HTTPS 443 when SSL is on. |
| Legacy API | The existing /cmdapi endpoint is unchanged and remains separate from this JSON command endpoint. |

## HTTP status mapping

| Status | Meaning | Response |
| --- | --- | --- |
| 200 | The command completed successfully. | ok is true and result contains the command output. |
| 400 | Malformed body, invalid parameters or unknown action. | ok is false with invalid_request, unknown_action or another command error. |
| 403 | The peer is not allowed by the configured API IP/MAC rules. | ok is false with unauthorized. |
| 413 | The request body exceeds 1 MiB. | ok is false with message_too_large. |
| 503 | The HTTP/HTTPS Control API is disabled. | ok is false, id is null and error.code is api_disabled. |
| 504 | An asynchronous device or hardware operation timed out. | ok is false with timeout. |
| 500 | The dispatcher failed internally or produced an invalid response. | ok is false with internal_error. |

## HTTP request example

POST /api/v1/command HTTP/1.1
Content-Type: application/json

{"protocol":2,"id":"mobile-1","action":"hello","params":{}}

{"protocol":2,"id":"mobile-1","ok":true,"result":{"device":"dimmer","api_port":5008}}

## Verification commands

Run the source-level contract tests from the Soleux App project root on a developer computer with Python 3. Each active device family has its own tests folder; the Dimmer command contract is verified with:

python -m unittest discover -s "Device Firmware\Dimmer\tests" -p "test_*.py" -v

After deploying the matching firmware files and restarting WebService, run this on the device to verify the live HTTP endpoint:

curl -sS -X POST http://127.0.0.1/api/v1/command \
  -H 'Content-Type: application/json' \
  -d '{"protocol":2,"id":"test-1","action":"hello","params":{}}'

Use https://127.0.0.1/api/v1/command when SSL is enabled. A successful response has ok: true and reports the calculated api_port.

Compatibility rule: The J: prefix is not accepted on the Control API or migrated base ports. Standard Windows paths use unprefixed JSON on base port + 3 and do not send AT commands or fall back to the AT-only base port. Adding the HTTP/HTTPS endpoint does not change existing legacy AT behavior.

# Permissions and security

| Permission | Allows |
| --- | --- |
| Public | hello only. |
| Read | Read identity, state, configuration summaries, telemetry and events. |
| Control | Change live input/output/dimmer state and execute schedules/scenarios. |
| Configure | Change mappings, channel configuration, schedules, automations and non-secret settings. |
| Admin | Accounts, access rules, network/security settings, backup/restore, firmware, reboot and destructive actions. |

The implemented HTTP/HTTPS command endpoint first requires api_enabled. When enabled, each device's existing API IP/MAC allow rules apply to the direct socket peer. Disabling filtering allows any reachable peer; it is different from disabling the API service. Forwarding headers are not trusted. Restrict HTTP to trusted local networks and use HTTPS for credentials, backup/restore, firmware, or remote/mobile access. Tokens and secrets must never be written to normal logs.

## Implemented service access switches

All five active firmware families persist the following independent Security-page switches. Both default to enabled. Changing either switch schedules a WebService restart after the current response is delivered.

| Security field | Behavior when disabled | Still available and recovery |
| --- | --- | --- |
| tcp_enabled | Closes and suppresses the base TCP, base + 1 and Control API TCP base + 3 listeners after restart. Existing TCP sessions close and the Windows app cannot connect. | UDP discovery, UDP heartbeat base + 2 and the web UI remain available. Re-enable TCP from the web Security page. |
| api_enabled | HTTP/HTTPS command routes return status 503 with id null and error.code api_disabled. Raw Control API TCP is not closed. | The normal web UI remains available. Re-enable API from the web Security page. |

Disabling API does not close Control API TCP; disabling TCP does. If both are disabled, the web Security page remains the recovery path. Discovery or heartbeat success therefore does not prove that device control is enabled.

{"protocol":2,"id":null,"ok":false,"error":{"code":"api_disabled","message":"HTTP/HTTPS API access is disabled."}}

## Implemented AC/DC Dimmer profile

The active Dimmer firmware accepts unprefixed protocol 2 JSON on base_port + 3 (normally 5008) and the identical command object through POST /api/v1/command over HTTP or HTTPS. The base port remains AT-only and rejects J-prefixed traffic.

All Dimmer input, output and mapping indices are zero-based. set_dimmer_level requires channel and an integer value from 0 through 100. set_output_state and restart_output use the same zero-based channel rule.

get_device_state returns requested set_pwm and live actual_pwm for every output, plus I/O state, PWM frequency, sensors, system and network data. Clients should poll this snapshot when a state-changing result reports pending: true.

The Dimmer also implements device identity, relay configuration, input/output configuration, mappings, hardware write, settings/security/system/schedule/automation/scenario/sequence pages, chunked backup/restore, paginated system logs and execute_page_action delete_system_logs.

Discovery advertises PORT, API_PORT, API_VER and CAPS. Heartbeat uses base_port + 2 and reports tcp_port, api_port and api_version. Both offsets are calculated from the configured base port.

# Common error catalogue

Every error uses the common error envelope. Commands may document additional codes.

| Code | Meaning |
| --- | --- |
| invalid_request | Envelope or JSON structure is invalid. |
| unknown_action | Action name is not recognized. |
| unsupported_command | Device recognizes the action but does not support it. |
| unsupported_protocol | No mutually supported protocol version exists. |
| authentication_required | A valid authenticated session is required. |
| authentication_failed | Credentials or token were rejected. |
| permission_denied | Session lacks the required permission. |
| invalid_parameter | A parameter has the wrong type, range or value. |
| invalid_channel | Input/output channel is outside the zero-based device range. |
| not_found | Requested resource or operation does not exist. |
| revision_conflict | expected_revision does not match current resource revision. |
| busy | A conflicting operation is active. |
| interlock | A safety or control interlock prevented the operation. |
| timeout | The operation did not complete within its allowed time. |
| rate_limited | Caller exceeded the configured request limit. |
| payload_too_large | Message or transfer exceeds advertised limits. |
| internal_error | Unexpected device-side failure; message must not expose secrets. |
| api_disabled | The HTTP/HTTPS Control API is disabled in device Security settings. |

# Implemented command catalogue

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| complete | boolean | Always | True after cleanup. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Open download session | Transfer identifier. |

Parameters

Implemented by: All active profiles    Operation: Admin

Close a download session and remove its temporary snapshot.

## 5.7  transfer_download_finish

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| data | base64 string | Always | Returned bytes. |
| offset | integer | Always | Echoed byte offset. |
| size | integer | Always | Total download size. |
| complete | boolean | Always | True when this chunk reaches the end. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Open download session | Transfer identifier. |
| offset | integer | Yes | Valid byte offset | First byte to read. |

Parameters

Implemented by: All active profiles    Operation: Admin

Read one base64-encoded data chunk from an open download session.

## 5.6  transfer_download_chunk

Command errors: internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| transfer_id | string | Always | Download-session identifier. |
| size | integer | Always | Snapshot size in bytes. |
| chunk_size | integer | Always | Maximum/recommended read size. |
| digest | string | When provided | Snapshot digest. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Parameters

Implemented by: All active profiles    Operation: Admin

Create a temporary settings-database snapshot and start a chunked download session.

## 5.5  settings_backup_download_begin

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| message | string | Always | Commit result description. |
| reboot_now | boolean | Always | True when firmware schedules an immediate reboot. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Completed upload session | Transfer to commit. |

Parameters

Implemented by: All active profiles    Operation: Admin

Install or restore a validated upload and report whether the device should reboot.

## 5.4  transfer_commit

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| complete | boolean | Always | True after size and file validation succeeds. |
| role | string | Always | Completed role. |
| digest | string | When provided | Calculated file digest. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Open upload session | Transfer identifier. |
| role | string | Yes | Completed file role | Role to finish. |

Parameters

Implemented by: All active profiles    Operation: Admin

Finish and validate one uploaded file role before commit.

## 5.3  transfer_upload_finish

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| received | integer | Always | Total bytes received for this role. |
| size | integer | Always | Expected total file size. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| transfer_id | string | Yes | Open upload session | Transfer identifier. |
| role | string | Yes | Role opened by begin | File role. |
| offset | integer | Yes | Exact next byte offset | Chunk byte offset. |
| data | base64 string | Yes | Within negotiated chunk size | Chunk bytes. |

Parameters

Implemented by: All active profiles    Operation: Admin

Append one base64-encoded byte chunk at the required upload offset.

## 5.2  transfer_upload_chunk

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| transfer_id | string | Always | Upload-session identifier. |
| role | string | Always | Accepted file role. |
| size | integer | Always | Expected total file size. |
| received | integer | Always | Bytes currently received. |
| chunk_size | integer | Always | Maximum/recommended chunk size. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| kind | string | Yes | Profile-supported transfer kind | Upload/commit workflow. |
| role | string | Yes | Kind-specific file role | Role of this file in the transfer. |
| filename | string | Yes | Safe base filename | Original file name. |
| size | integer | Yes | Profile size limit | Expected file size in bytes. |
| transfer_id | string | No | Existing multi-file transfer | Attach another role to an existing transfer. |

Parameters

Implemented by: All active profiles    Operation: Admin

Create or extend a chunked upload session for settings, credentials or firmware files supported by the profile.

## 5.1  transfer_upload_begin

# 5. Backup restore and transfers

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| result | object | Always | Action acknowledgment, test result or refreshed page data. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| page_action | string | Yes | Profile-specific implemented action | Operation to execute. |
| datetime | string | Conditional | YYYY-MM-DD HH:MM:SS | Client/manual time for supported time actions. |
| server | string | Conditional | Host name or IP | NTP server to test. |
| log_page_size | integer | No | delete_system_logs | Requested size of the refreshed system-log page. |
| action-specific fields | mixed | Conditional | Dimmer sequence/scenario rewrites | Additional values required by the selected page action. |

Parameters

Implemented by: All active profiles    Operation: Configure or admin

Execute an implemented page operation such as time synchronization, NTP test, log deletion or reboot.

## 4.4  execute_page_action

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| page | object | Always | Updated page/row collection after the mutation. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| page | string | Yes | Profile-specific page name | Page containing the row collection. |
| section | string | Yes | Supported row section | Row collection to mutate. |
| operation | string | Yes | add, update or delete | Requested row operation. |
| id | integer | Update/delete | Database row ID | Existing row identifier. |
| values | object | Add/update | Section-specific fields | Row values to validate and persist. |

Parameters

Implemented by: All active profiles    Operation: Configure or admin

Add, update or delete one row in a supported schedule, automation or security table.

## 4.3  mutate_page_row

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| page | object | Always | Updated page or section result. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| page | string | Yes | Profile-specific page name | Page containing the section. |
| section | string | Yes | Supported section name | Section to update. |
| values | object | Yes | Section-specific fields | Values to validate and persist. |

Parameters

Implemented by: All active profiles    Operation: Configure or admin

Save one supported section of a device configuration page.

## 4.2  set_page_configuration

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| page | object | Always | Page-specific sections, fields, columns, rows and optional pagination. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| page | string | Yes | Profile-specific page name | Page to read. |
| log_page | integer | No | System page; default 1 | One-based system-log page. |
| log_page_size | integer | No | System page; profile limits | Number of log rows per page. |

Parameters

Implemented by: All active profiles    Operation: Read

Read a supported UI/configuration page using the device's existing page schema.

## 4.1  get_page_configuration

# 4. Configuration pages

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| energy_history | boolean | Always | True for an energy-history result. |
| parameter | integer | Always | Echoed measurement selector. |
| unit | string | Always | Measurement unit. |
| start_date | date | Always | Echoed start date. |
| end_date | date | Always | Echoed end date. |
| points | object[] | Always | Timestamp/value samples, limited by down-sampling. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| parameter | integer | Yes | 0 voltage, 1 current, 2 power, 3 PF, 4 VA, 5 energy | Measurement selector. |
| start_date | date | Yes | YYYY-MM-DD | Inclusive start date. |
| end_date | date | Yes | YYYY-MM-DD; maximum 365 days | Inclusive end date. |

Parameters

Implemented by: PDU Energy Meter    Operation: Read

Read down-sampled historical Energy Meter points for one measurement and date range.

## 3.4  get_energy_history

Command errors: internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| energy | object | Always | Energy measurement object. |
| energy.voltage_v | number or null | Always | Voltage reading. |
| energy.current_a | number or null | Always | Current reading. |
| energy.active_power_w | number or null | Always | Active power. |
| energy.apparent_power_va | number or null | Always | Apparent power. |
| energy.power_factor | number or null | Always | Power factor. |
| energy.energy_kwh | number or null | Always | Accumulated energy. |
| energy.available | boolean | Always | True when every required reading is available. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Parameters

Implemented by: PDU Energy Meter    Operation: Read

Read the current PDU Energy Meter electrical measurements without converting missing readings to zero.

## 3.3  get_energy_state

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| pwm_frequency | integer | Always | Saved frequency-mode value. |
| saved | boolean | Always | True after persistence and publish. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| value | integer | Yes | AC: 0 or 1; DC: 1, 2 or 3 | Frequency mode accepted by the installed Dimmer type. |

Parameters

Implemented by: AC/DC Dimmer    Operation: Configure

Save and apply the AC/DC Dimmer frequency-mode value.

## 3.2  set_dimmer_frequency

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Controlled Dimmer channel. |
| requested_pwm | integer | Always | Requested PWM percentage. |
| actual_pwm | integer | Always | Latest known hardware PWM percentage. |
| pending | boolean | Always | True while actual PWM differs from requested PWM. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to output_count - 1 | Zero-based Dimmer output channel. |
| value | integer | Yes | 0 through 100 | Requested PWM percentage. |

Parameters

Implemented by: AC/DC Dimmer    Operation: Control

Set the PWM brightness percentage for one Dimmer output channel.

## 3.1  set_dimmer_level

# 3. Dimmer and energy control

Command errors: internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| hardware_write_started | boolean | Relay/PDU profiles | True when the asynchronous write begins. |
| hardware_write_complete | boolean | Dimmer | True when the Dimmer write request completes. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Parameters

Implemented by: Relay, Dimmer, PDU Energy, PDU 10 kW    Operation: Configure

Publish saved mappings and output-delay configuration to the hardware-control services.

## 2.8  write_hardware

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| saved | boolean | Always | True after the database update was accepted. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| input | integer | Yes | 0 to input_count - 1 | Input side of the mapping. |
| output | integer | Yes | 0 to output_count - 1 | Output side of the mapping. |
| code | integer | Yes | Profile mapping-code range | Existing device mapping behavior code. |

Parameters

Implemented by: Relay, Dimmer, PDU Energy, PDU 10 kW    Operation: Configure

Save one zero-based input-to-output mapping using the current numeric behavior code.

## 2.7  set_mapping

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| output | object | Always | Updated output configuration object, returned directly or as the command result. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to output_count - 1 | Output channel to update. |
| name | string | No | Profile length rules | New display name. |
| enabled | boolean | No | Profile dependent | Whether the output is enabled. |
| delay/runtime/interlock fields | mixed | No | Profile dependent | On/off delay, runtime, restart-disable, turn-off-disable and Dimmer PWM fields. |

Parameters

Implemented by: All active profiles    Operation: Configure

Save the editable configuration for one output; supported fields vary by device profile.

## 2.6  set_output_configuration

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Restarted output channel. |
| accepted | boolean | Always | True when the restart was queued. |
| operation_id | string | Always | Identifier for the asynchronous restart. |
| off_time_ms | integer | Dimmer | Configured restart off-time used by the Dimmer path. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to output_count - 1 | Output channel to restart. |
| source | string | No | Default Control API | Audit/source description. |

Parameters

Implemented by: All active profiles    Operation: Control

Start the existing output restart operation for one zero-based output channel.

## 2.5  restart_output

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Controlled output channel. |
| requested_state | boolean | Always | Requested logical state. |
| actual_state | boolean | Always | Latest known hardware state. |
| pending | boolean | Always | True until hardware feedback matches the request. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to output_count - 1 | Zero-based output channel. |
| state | boolean | Yes | true or false | Requested output state. |
| source | string | No | Default Control API | Audit/source description when supported. |

Parameters

Implemented by: All active profiles    Operation: Control

Request one output to turn on or off while enforcing manual-override and configured interlocks.

## 2.4  set_output_state

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| input | object | Always | Updated input configuration object, returned directly or as the command result. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to input_count - 1 | Input channel to update. |
| name | string | No | Profile length rules | New display name. |
| enabled | boolean | No | true or false | Whether the input is enabled. |

Parameters

Implemented by: Relay, Dimmer, PDU Energy, PDU 10 kW    Operation: Configure

Save the editable configuration for one zero-based input channel.

## 2.3  set_input_configuration

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Accepted input channel. |
| state | boolean | Always | Requested input state. |
| accepted | boolean | Always | True when queued/published. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to input_count - 1 | Zero-based input channel in the profile input space. |
| state | boolean | Yes | true or false | Requested virtual input state. |
| source | string | No | Default Control API; max 64 chars | Audit/source description. |

Parameters

Implemented by: Relay, Dimmer, PDU Energy, PDU 10 kW    Operation: Control

Compatibility alias for input-state injection on profiles that expose virtual input control.

## 2.2  set_virtual_input_state

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| channel | integer | Always | Accepted input channel. |
| state | boolean | Always | Requested input state. |
| accepted | boolean | Always | True when queued/published. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| channel | integer | Yes | 0 to input_count - 1 | Zero-based input channel. |
| state | boolean | Yes | true or false | Requested input state. |
| source | string | No | Default Control API; max 64 chars | Audit/source description. |

Parameters

Implemented by: Relay, Dimmer, PDU Energy, PDU 10 kW    Operation: Control

Inject a zero-based input state through the same control path used by the device input service.

## 2.1  set_input_state

# 2. Input output and mapping control

Command errors: internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| input_count | integer | Always | Installed input count. |
| output_count | integer | Always | Installed output count. |
| inputs | object[] | Always | Input names, enable flags and profile fields. |
| outputs | object[] | Always | Output names, delays, interlocks and profile fields. |
| mapping | object[] | When supported | Zero-based input/output mapping data. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Parameters

Implemented by: All active profiles    Operation: Read

Read the installed input/output configuration and input-to-output mapping matrix.

## 1.4  get_relay_configuration

Command errors: internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| revision | integer | When available | Snapshot revision/timestamp value. |
| captured_at | datetime | When available | Snapshot capture time. |
| inputs | object[] | Profiles with inputs | Current zero-based input states. |
| outputs | object[] | Always | Output names, states and device-specific live values. |
| sensors | object[] | When available | Temperature and environmental readings. |
| system | object | Always | Time, uptime, memory, CPU and temperature data. |
| network | object | Always | Current LAN/Wi-Fi addresses and SSID. |
| energy | object | PDU Energy Meter | Current electrical measurements. |
| manual_override_active | boolean | PDU profiles | Current manual override state. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Parameters

Implemented by: All active profiles    Operation: Read

Read the complete live snapshot used by desktop and mobile clients for synchronization.

## 1.3  get_device_state

Command errors: internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| device_type | string | Always | Device family/profile. |
| model | string | Device dependent | Hardware model or product name. |
| serial_number | string | Always | Configured device serial number. |
| firmware_version | string | When available | Installed firmware version. |
| build_number | string | When available | Installed build identifier. |
| release_date | string | When available | Firmware release/build date. |
| network | object | Always | Ethernet and Wi-Fi MAC addresses. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Parameters

Implemented by: All active profiles    Operation: Read

Read stable device identity, firmware/build information and Ethernet/Wi-Fi MAC addresses.

## 1.2  get_device_info

Command errors: invalid_request, internal_error

| Field | Type | Presence | Description |
| --- | --- | --- | --- |
| protocol | integer | Always | Current Control API protocol number, 2. |
| device | string | Always | Device profile identifier. |
| name | string | Always | Configured device name. |
| input_count | integer | Always | Installed physical/control input count. |
| virtual_input_count | integer | Always | Installed virtual input count, or zero. |
| output_count | integer | Always | Installed controllable output count. |
| api_port | integer | Always | Calculated Control API TCP port. |
| capabilities | string[] | Always | Advertised action and feature capabilities. |

Expected result

| Field | Type | Required | Constraints or default | Description |
| --- | --- | --- | --- | --- |
| (none) | - | - | - | This action has no action-specific parameters. |

Parameters

Implemented by: All active profiles    Operation: Public read

Negotiate protocol 2 and return the device identity, installed channel counts, API port and advertised capabilities.

## 1.1  hello

# 1. Device session and state

The catalogue intentionally excludes target or proposed actions that do not appear in the active firmware dispatcher and hello capability data.

| Action | Implemented by | Parameters | Result or purpose |
| --- | --- | --- | --- |
| hello | All active profiles | None | Protocol, identity and capability list. |
| get_device_info | All active profiles | None | Stable identity, firmware and network information. |
| get_device_state | All active profiles | None | Live inputs, outputs, PWM, sensors, system and network snapshot. |
| get_relay_configuration | All active profiles | None | Input, output and mapping configuration. |
| set_input_state | Relay, Dimmer, PDU Energy, PDU 10 kW | channel, state, source | Inject a physical input state. |
| set_virtual_input_state | Relay, Dimmer, PDU Energy, PDU 10 kW | channel, state, source | Inject a virtual input state. |
| set_input_configuration | Relay, Dimmer, PDU Energy, PDU 10 kW | channel, name, enabled | Save one input configuration. |
| set_output_state | All active profiles | channel, state, source | Request an output on or off state. |
| restart_output | All active profiles | channel, source | Restart one output. |
| set_output_configuration | All active profiles | Device-specific output fields | Save one output configuration. |
| set_mapping | Relay, Dimmer, PDU Energy, PDU 10 kW | input, output, code | Save one input-to-output mapping. |
| write_hardware | Relay, Dimmer, PDU Energy, PDU 10 kW | None | Publish saved mappings and output delays to hardware services. |
| set_dimmer_level | AC/DC Dimmer | channel, value (0-100) | Set one PWM brightness level. |
| set_dimmer_frequency | AC/DC Dimmer | value | Save and apply the dimmer frequency mode. |
| get_energy_state | PDU Energy Meter | None | Read current aggregate energy measurements. |
| get_energy_history | PDU Energy Meter | parameter, start_date, end_date | Read stored energy history. |
| get_page_configuration | All active profiles | page, optional pagination | Read a supported configuration page. |
| set_page_configuration | All active profiles | page, section, values | Save a supported configuration page section. |
| mutate_page_row | All active profiles | page, section, operation, row_id, values | Create, update or delete a supported page row. |
| execute_page_action | All active profiles | page_action and action-specific fields | Execute supported system, time, log and reboot actions. |
| transfer_upload_begin | All active profiles | kind, role, filename, size | Start a chunked upload session. |
| transfer_upload_chunk | All active profiles | transfer_id, role, offset, data | Append one upload chunk. |
| transfer_upload_finish | All active profiles | transfer_id, role | Finish and validate an upload. |
| transfer_commit | All active profiles | transfer_id | Commit a validated upload or restore. |
| settings_backup_download_begin | All active profiles | None | Start a settings-backup download. |
| transfer_download_chunk | All active profiles | transfer_id, offset | Read one download chunk. |
| transfer_download_finish | All active profiles | transfer_id | Finish a download session. |

This catalogue contains only request actions implemented by the active protocol-2 firmware profiles. Device-specific actions are listed with their applicable profile; clients should still use the hello capability list before sending a device-specific action.

# Shared data schemas

These compact definitions identify the fields expected in repeated result objects. A later JSON Schema/OpenAPI file can make them machine-validatable without changing the command contract.

| Type | Required/core fields |
| --- | --- |
| DeviceInfo | device_id, name, device_type, model, serial_number, firmware_version, hardware_version |
| Topology | input_count, virtual_input_count, output_count, sensor_count |
| InputState | kind, channel, name, enabled, state, mode, active_level, debounce_ms, changed_at |
| OutputState | channel, name, enabled, state, pending, requested_level?, actual_level?, configuration?, changed_at |
| OutputConfiguration | channel, name, enabled, delays/runtimes in ms, initial_state, restart_off_time_ms |
| Mapping | input, output, behavior, legacy_code; codes: 0 none, 1 on, 2 off, 3 toggle, 4 continuous_on, 5 continuous_off |
| Schedule | id, revision, name, enabled, type, timing, actions, last_run, next_run |
| Schedule timing | one_time: datetime; recurring: time, weekdays[0=Monday..6=Sunday], optional start/end dates |
| Action | action plus params; only capability-advertised control actions are permitted |
| Automation | id, revision, name, enabled, type, trigger configuration and actions |
| Scenario | id, revision, name, default_transition_ms and targets[channel,state,level] |
| Sequence | id, revision, name, steps[scenario_id or actions, hold_ms], loop_count |
| TemperatureReading | sensor_id, name, value_c, status, timestamp |
| ChannelEnergy | channel, voltage_v?, current_a?, active_power_w?, apparent_power_va?, power_factor?, energy_wh?, timestamp |
| AccessRule | id, service, kind, value, name, enabled, revision |
| Fault | fault_id, severity, code, message, active, first_seen, last_seen |

## Configuration section schemas

| Section | Fields |
| --- | --- |
| general | name, location?, description?, language?, physical_input_count (read-only), output_count (read-only) |
| account | username, password (write-only), session_timeout_s, password_change_required |
| lan | mode[dhcp¦static], address, subnet_mask, gateway, dns_servers[], control_port = legacy_port + 3 (default 5008), legacy_port default 5005 |
| wifi | enabled, mode[dhcp¦static], ssid, password (write-only), address, subnet_mask, gateway, dns_servers[] |
| time | timezone, sync_mode[manual¦ntp], ntp_server, sync_interval_s |
| temperature | enabled, low_warning_c, high_warning_c, control/hysteresis fields where supported |
| tcp_access | tcp_enabled, legacy_port, allow_filter_enabled; tcp_enabled controls legacy_port, legacy_port + 1 and Control API TCP legacy_port + 3; rules are managed separately |
| api_access | api_enabled, IP/MAC allow rules, HTTP enabled, HTTPS enabled, authentication target, rate-limit target; api_enabled controls HTTP/HTTPS command endpoints only |
| mqtt | enabled, host, port, tls, username, password(write-only), client_id, base_topic, qos |
| tls | enabled, certificate metadata, key_present, minimum_tls_version; key material is write-only via upload |
| maintenance | log_level, automatic_backup settings, retention, update channel where supported |

# Device events

Events are unsolicited messages on subscribed TCP/WebSocket sessions or SSE streams. They use protocol, event, subscription_id and data fields; they do not contain ok or result.

{"protocol":3,"event":"output_state_changed","subscription_id":"sub-7","data":{"channel":0,"previous_state":false,"state":true,"pending":false,"source":"windows-app","revision":312,"timestamp":"2026-08-31T10:20:30+00:00"}}

| Event | Expected data fields |
| --- | --- |
| output_state_changed | channel, previous_state, state, pending, source, revision, timestamp |
| output_level_changed | channel, requested_level, actual_level, transitioning, source, revision, timestamp |
| input_state_changed | kind, channel, previous_state, state, source, revision, timestamp |
| mapping_changed | input, output, behavior, legacy_code, revision, timestamp |
| temperature_changed | sensor_id, value_c, status, timestamp |
| energy_changed | summary and/or channel readings, timestamp |
| schedule_executed | schedule_id, results, success, timestamp |
| automation_executed | automation_id, trigger, results, success, timestamp |
| sequence_state_changed | sequence_id, operation_id, state, step, loop, timestamp |
| operation_progress | operation_id, kind, stage, progress_percent, message, timestamp |
| device_fault | fault_id, severity, code, message, active, timestamp |
| configuration_changed | section, revision, source, restart_required, timestamp |
| device_rebooting | reason, reboot_in_ms, timestamp |

Synchronization: Clients must treat events as incremental updates. If revisions are skipped, reconnecting clients should call get_device_state to rebuild a complete state snapshot.

# Migration and acceptance criteria

- Migrated Relay Module, AC/DC Dimmer and PDU firmware exposes its implemented protocol 2 command subset on configured base port + 3 (5008 by default).

- Each migrated firmware family exposes the same implemented commands through POST /api/v1/command on HTTP or HTTPS.

- Discovery and heartbeat identity report the calculated Control API port so clients do not hardcode 5008.

- The standard Windows paths for all five migrated families use the JSON Control API and do not send AT commands or use the base port.

- Each configured base port remains AT-only for existing third-party or service integrations.

- Each migrated screen is tested against Relay Module, AC/DC Dimmer, PDU Energy Meter, PDU 10 kW and PDU V1 capabilities as applicable.

- Every active firmware family creates tcp_enabled and api_enabled automatically during database migration, defaults both to enabled, and persists later changes.

- With tcp_enabled false, base TCP, base + 1 and base + 3 are closed after restart while discovery, heartbeat and the web UI remain active.

- With api_enabled false, HTTP/HTTPS command routes return 503 api_disabled while raw Control API TCP and the web UI remain available.

- The mobile application uses the Control API envelope and relies on hello/capability data instead of a fixed port or device-model assumptions.

- Automated tests verify TCP framing, invalid JSON, HTTP status mapping, 1 MiB size limits, allow rules, index bounds, concurrency and reconnection.

The implemented-only catalogue must be updated whenever a new action is added to firmware dispatch, hello capabilities, the Windows/mobile client path and its automated contract tests.